INTERIM report 1

EGYPT

Feasibility Study For The Establishment Of An Energy Efficiency Fund

BMZ No. 2019 69146

Presented to:

Ministry of Electricity and Renewable Energy / represented by KfW

June 2023

TABLE OF CONTENTS

1	Executive Summary	1

2	Market assessment – methodology and approach	3

2.1	Methodology and approach	3

2.2	Assessment criteria	3

2.3	EE measures, technologies, appliances and services	5

3	Background and overview	7

3.1	Energy consumption profile in Egypt	7

3.2	EE targets and plans	8

3.3	EE standards and regulations	9

3.4	Key programs	13

4	Market demand assessment	15

4.1	Residential sector	15

4.2	Commercial sector	26

4.3	Public sector	35

5	Market Supply Side assessment	44

5.1	EE Market Actors	44

5.2	Available financing Mechanism	51

5.3	Gap analysis and Identified needs for technical assistance	53

5.4	Lessons learned, cooperation opportunities with other schemes and institutions	53

6	Pipeline development	56

6.1	Drivers	56

6.2	Barriers	56

6.3	Selection Criteria	57

LIST OF FIGURES

Figure 1: Energy consumption % across different segments (percentage are rounded), Source EEHC	7

Figure 2: Development of energy consumption in different sectors: EEHC	8

Figure 3: Institutional framework for EE	10

Figure 4: Development of residential sector energy consumption. Source: EEHC, 2023	15

Figure 5: Consumption categories distribution	16

Figure 6: Climatic areas in Egypt, Source: Ali. K Abdel-Rahman et al	16

Figure 7: Residential electricity consumption categories, Source: The world bank 2017	17

Figure 8: Specific electricity consumption	18

Figure 9: Shares of different types on cooling appliances	20

Figure 10: Electricity consumption before replacing AC	25

Figure 11: Electricity consumption after replacing AC	25

Figure 12: Electricity consumption breakdown	28

Figure 13: Electricity consumption for surveyed hotels	28

Figure 14: Chilled water and electricity intensity	32

Figure 15: Electricity consumption percentage by application	32

Figure 16: Public buildings consumption trend	35

Figure 17: Energy consumption profile in public universities	38

Figure 18: Energy consumption profile in public administrative buildings	41

Figure 19: Loan guarantee program design, Source UNIDO	45

Figure 19: Sold electricity % by DISCOs, Source: EEHC	48

Figure 20: Shares of NO. of subscriber, Source: EEHC	49

Figure 21: Sold and purchased electricity by each DISCOs	49

LIST OF TABLES

Es konnten keine Einträge für ein Abbildungsverzeichnis gefunden werden.

ACRONYMS

| AEC | Average Energy Consumption |
| --- | --- |
| AFD | French Development Agency |
| AfDB | African Development Bank |
| AHU | Air Handling Units |
| ASA | Accountability State Authority |
| BAT | Best Available Technology |
| BEP | Building Energy Performance |
| BMS | Building Management System |
| BMZ | German Federal Ministry for Economic Cooperation and Development |
| BMZ | German Federal Ministry for Economic Cooperation and Development |
| BoD | Board of directors |
| BREF | Best available technology Reference Document |
| BS | Backstopper |
| CAPEX | Capital Expenditures |
| CAPMAS | Central Agency for Public Mobilization and Statistics |
| CBE | Central Bank of Egypt |
| CEEU | Central Energy Efficiency Unit |
| CO2 | Carbon Dioxide |
| CSO | Civil Society Organisation |
| DCR | Debt Coverage Ratio |
| DD | Due Diligence |
| DISCO | Electricity Distribution Company |
| DSCR | Debt Service Coverage Ratio |
| DSM | Demand side management |
| E&S | Environmental and Social |
| EA | Energy Audit |
| EBRD | European Bank for Reconstruction and Development |
| EC | European Commission |
| ECM | Energy Conservation Measure |
| ED | Energy Department |
| EE | Energy Efficiency |
| EECCPD | Energy Efficiency and Climate Change Project Department |
| EEF | Energy Efficiency Fund |
| EEHC | Egyptian Electricity Holding Company |
| EEIF | Energy Efficiency Institutional Framework |
| EGP | Egyptian Pound |
| EIB | European Investment Bank |
| EgyptERA | Electricity Utility and Consumer Protection Regulatory Agency |
| EM | Energy Manager |
| EMS | Energy Management System |
| EPB | Energy Performance of Buildings |
| EPC | Energy Performance Contract |
| ESCO | Energy Service Company |
| ESG | Environmental Social Governance |
| ESMS | Environmental and Social Management System |
| ESRM | Environmental & Social Risk Management |
| EU | European Union |
| FI | Financial Institution |
| FIN | Financial |
| FRA | Financial Regulatory Authority |
| FS | Feasibility Study |
| FY | Fiscal Year |
| GAIFZ | General Authority for Investment and Free Zones |
| GEF | Global Environment Facility |
| GEFF | Green Economy Financing Facility |
| GFA | GFA Consulting Group GmbH |
| GHG | Greenhouse Gas |
| GIZ | Deutsche Gesellschaft für Internationale Zusammenarbeit GmbH |
| GoE | Government of Egypt |
| HV | High Voltage |
| HVAC | Heating, Ventilation and Air-Conditioning |
| IFC | International Finance Corporation |
| IFI | International Financial Institution |
| INT | International |
| ISES | Integrated Sustainable Energy Strategy |
| IT | Information technology |
| JV | Joint Venture |
| KfW | KfW Development Bank |
| KPI | Key Performance Indicator |
| LED | Light Emitting Diode |
| LOC | Local |
| M&E | Monitoring and evaluation |
| M&V | Measurement and Verification |
| MERR | Monitoring, Evaluation, Reporting and Risk Management |
| MISR | Banque Misr |
| MoERE | Ministry of Electricity and Renewable Energy |
| MoF | Ministry of Finance |
| MoH | Ministry of Health |
| MoIC | Ministry of International Cooperation |
| MoPED | Ministry of Planning and Economic Development |
| MoT | Ministry of Tourism |
| MS | Microsoft |
| MV | Medium Voltage |
| MRV | Measurement, reporting and verification, |
| NBE | National Bank of Egypt |
| NCCS | National Climate Change Strategy |
| NDC | National Determined Contribution |
| NEEAP | National Energy Efficiency Action Plan |
| NGO | Non-Governmental Organization |
| PA | Project Assessment |
| PAC | Project Advisory Committee |
| PIU | Project Implementing Unit (consisting of PEA and Management Consultant) |
| PV | Photovoltaic |
| Q&A | Questions and Answers |
| RE | Renewable Energy |
| REEF | Revolving Energy Efficiency Fund |
| RES | Renewable Energy Sources |
| RfP | Request for Proposal |
| ROI | Return on Investment |
| SDG | Sustainable Development Goals |
| SDS | Sustainable Development Strategy: Egypt Vision 2030 |
| SEU | Significant Energy User |
| SME | Small and midsize enterprises |
| SQ | Sequence |
| SU | Sectoral Unit |
| SWOT | Strengths, Weaknesses, Opportunities, Threats |
| TA | Technical Assistance |
| TCFD | Task Force on Climate-Related Financial Disclosures |
| TEC | Total Energy Consumption |
| TECH | Technical |
| TL | Team Leader |
| ToR | Terms of Reference |
| ToU | Time of Use (tariff) |
| TSFE | The Sovereign Fund of Egypt |
| TSO | Transmission System Operator |
| UAE | United Arab Emirates |
| UNDP | United Nations Development Programme |
| UNIDO | United Nations Industrial Development Organization |
| USD | US-Dollar |
| VRV | Variable Refrigerant Volume |
| VSD | Variable Speed Drivers |
| WB | World Bank |

# Executive Summary

xxx

# Market assessment – methodology and approach

## Methodology and approach

The inception report proposed the market assessment methodology, however, the team encountered several challenges with data collection: lack of available data and significant delays in receiving data. Thus, the market assessment methodology was adjusted to accommodate the new situation.

Based on the data collected, the residential sector has undergone changes in its segmentation. The sector is now segmented according to consumption groups. In the public sector, one segment (kindergarten) was removed due to insufficient data. In the commercial sector, two segments, namely hotels and office buildings, were investigated, as for other types of buildings there was no available consumption data and no energy audits to establish the energy consumption profile. The table below provides a summary of the segments.

Table 1: Target Sectors and segments

| TARGET SECTORS AND SEGMENTS |  |  |
| --- | --- | --- |
| RESIDENTIAL | PUBLIC | COMMERCIAL |
| 0-50 | Hospitals | Office buildings |
| 51-100 | Administrative buildings | Hotels & resorts |
| 0-200 | Universities |  |
| 201-350 | Schools |  |
| 351-650 |  |  |
| 0-1000 |  |  |
| more than 1000 |  |  |

The following methodology is applied for market analysis with the aim to recommend the most suitable sectors and segments for the future operation of the Egyptian Energy Efficiency Fund (EE Fund) based on selection criteria.

## Assessment criteria

The suggested criteria are to measure the savings potential, ease of EE implementation, and viability. To assess each criterion, several variables are chosen carefully. The table below summarizes the proposed criteria and variables to assess the sectors and segments.

Table 2, Selected indicators and sub-indicators for market assessment

| CRITERIA | VARIABLES |
| --- | --- |
| SAVINGS POTENTIAL ASSESSMENT | - Total (TEC) energy consumption per sector and segment - Average energy consumption (AEC) per facility and segment according to the appropriate benchmark if available. |
| EASE OF EE IMPLEMENTATION | - Most significant energy users - List of potential ECMs and appliances |
| FEASIBILITY | - Potential monetary savings, energy savings in kWh, as well as primary energy savings, GHG savings, and payback period |

### Criterion 1: Savings potential assessment

This criterion identifies sectors and their segment(s) with the highest energy saving potential. In order to do this, the collection of the available input data was collected such as: the total energy consumption per sector and segment, the number of facilities per segment if available, and their operational mode (e.g., operation hours). Based on this input data the average energy consumption per facility or any other appropriate KPI was calculated. By comparing the current energy consumption (baseline energy consumption) with a benchmark, the potential savings after EE renovation in energy (kWh) and emissions savings (CO2) are estimated.

The assessment will identify the most Significant Energy Users (SEUs) within the sectors as well as the biggest energy and CO2 savings potential.

- Variable One: Total and average energy consumption in each sector and segment.

This input-data was collected from CAPMAS, the annual reports of EEHC and with the support of EECCPD and the Sectoral Units (SUs).

- Variable Two: Average energy consumption per segment and facility in the respective segment, the average energy consumption per square meter of the facility (kWh/m2) in the respective segment (e.g., hospitals, universities, schools, etc.).

This variable will provide more in-depth knowledge about energy consumption in the segments (e.g., hospitals, universities, schools, etc.). Moreover, it will help to compare the current energy consumption with benchmarks to assess the energy and CO2 savings potential.

### Criterion 2: Implementation of EE solution

For the successful start of the EE Fund operation, it is important to understand the maturity level of the market for the implementation of EE solutions. Some EE solutions, that require additional systems modifications or behavioral changes, are more difficult to be accepted in the market.

After identification of sectors and segment with the highest energy and CO2 savings potential, the most significant energy user (e.g., ECM, appliance, EE service, etc.) in each segment are pinpointed (e.g., HVAC in the commercial sector, appliances in the residential sector…). A list with pre-selected EE solutions (e.g., ECMs, EE services, technologies, appliances) is elaborated and further analyzed. As a last working step of this assessment, the likelihood of the demand for EE solutions is assessed or assumed.

Assessment of the applicability of the potential EE solutions that could decrease energy consumption in the pre-selected sectors and segments.

- Variable Three: Most Significant Energy Users (applications, e.g., steam system, appliances, etc.)

Collection of data about the applications and equipment that consumes significant energy, such as chillers, boilers, appliances, etc. This data is collected through energy audits, research, interviews, case studies, and literature.

- Variable Four: Potential energy conservation measures (ECMs)

Data about the most applicable EE solutions that could be implemented are collected and analyzed and scenarios with combination of ECMs are defined. A list of potential EE solutions in each target sub-sector will be generated. The data is collected from documents such as energy audits, BAT (Best Available Technology) and BREF (BAT reference document).

### Criterion 3: Feasibility (FIN and TECH) of the ECMs

This is to examine how feasible (financially and technically) the ECMs, short listed from assessing Criterion 2, are to be implemented in the segments.

Only technical solutions for which there is experience and knowledge in the market are proposed. Financial feasibility will be examined based on the current energy price for each sector and segment, respectively. KPIs such as simple payback, financial savings, GHG savings, investment/savings ratio (EGP per m2/kWh), etc., will be determined and assessed.

Assessment of the applicability of the pre-selected most efficient EE solutions that lead to reduced energy consumption in the selected sectors and segments.

- Variable Five: Availability of and experience on the market for the implementation of pre-selected ECMs

To apply EE solutions short listed after the analysis from the criterion above, data related to the availability of the EE solution and experienced companies installing these will be gathered and analyzed. For example, the installation of heat pumps would be a promising ECM that could be easily implemented; however, this ECM is not widely popular in Egypt. The data collection source is based on interviews and case studies.

- Variable Six: Monetary savings after ECMs implementation, GHG savings, payback, CAPEX, ROI, DSCR

This variable will consider the cost of energy supplied and potential monetary savings. The higher the savings the higher the motivation to implement ECMs. Moreover, data about necessary CAPEX per ECMs is collected or estimated based on research and available studies, thus the payback period will be calculated. The emission savings and the selected KPIs will be calculated under this sub-indicator.

By collecting, processing, and analyzing data regarding the energy consumption in each sector and segment, a consumption pattern in each sector and segment is estimated.

As a next step, the applications and available and applicable EE solutions to reduce most efficiently energy consumption are identified and used to select the most suitable sector and segment to be targeted by the EE Fund.

## EE measures, technologies, appliances and services

The table below provides an overview of EE measures, technologies, appliances and services.

Table 3: Potential ECMs, appliances and services by target sector

| POTENTIAL ECMs, APPLIANCES AND SERVICES |  |  |
| --- | --- | --- |
| RESIDENTIAL | PUBLIC | COMMERCIAL |
| Air-conditioners | RE solutions | RE solutions |
| Refrigerators | EE building retrofit | EE building retrofit |
| Water heaters | BMS/EMS | BMS/EMS |
| Efficient indoor lighting | Smart metering | Smart metering |
|  | Water efficiency | Water efficiency |
|  | Street lighting | Outdoor lighting |
|  | Water efficiency | Water efficiency |
|  | Operation & maintenance | Operation & maintenance |
|  | Facility management | Facility management |
|  | Measurement & Verification | Measurement & Verification |
|  | Energy audits | Energy audits |

During the market assessment, the use of previously applied ECMs solutions proved to be crucial in providing valuable insights into costs and potential savings. This data was instrumental in estimating KPIs and determining the size of the potential market.

Diverse measures that have been identified and grouped into packages are presented in the table below. Two levels of EE renovation for facilities are proposed, namely: simple and deep level, presenting scenarios with combination of measures from these packages.

Table 4. Technologies, appliances, measures and services grouped into packages

| TECHNOLOGIES / APPLIANCES AND SERVICES INCLUDED INTO PACKAGES |  |
| --- | --- |
| LIGHTING PACKAGE | - Indoor lighting (LED) - Skylight retrofit - Fibre optics retrofit |
| HVAC PACKAGE | - Replacement of old / inefficient systems - Installation of VRV systems - Replacement of old / inefficient AHU - Installation of heat recovery equipment - Installation of VSD on AHU - Installation of VSD on chilled / cooling water pumps / cooling towers - Measures recommended by EA |
| MOTORS PACKAGE | - Replacement of old / inefficient motors - Installation of VSD - Measures recommended by EA |
| INSULATION PACKAGE | - Replacement of single glazed windows - Insulation painting - Installation of shadings - Measures recommended by the energy audit |
| RE / SMART SYSTEM | - PV system - Solar thermal system - Installation of BMS/EMS - Measures recommended by EA |
| NON-EE PACKAGE | - Measures recommended by the energy audit - Measures that are required (e.g., painting the façade after the insulation works) but do not lead to additional energy savings |

# Background and overview

Climate change will have a significant impact on Egypt, resulting in extreme weather conditions like droughts, floods, and high temperatures. This will lead to substantial economic losses. The IPCC has predicted that the sea level will rise by one meter by the year 2100, causing the Nile delta, Northern coast, and Sinai to sink. Additionally, groundwater salinization will spread to new areas, reducing the availability of fresh water for irrigation and drinking.

It is widely acknowledged that carbon emissions are a significant contributor to climate change. According to statistics from 2015, Egypt's total CO2 emissions amounted to 325,614 thousand tons equivalent. The majority of these emissions, approximately 73%, are attributed to CO2, while CH4 accounts for 12.7%. Notably, the energy sector is responsible for the largest share of emissions, making up about 64.5% of the total. Egypt's electricity consumption has been growing rapidly, with an annual rate of approximately 6.2%, making it one of the highest in the world. To support the global fight against climate change, it is crucial for Egypt to take measures to reduce its energy consumption.

## Energy consumption profile in Egypt

Based on the EEHC's annual report for 2021/2022, the residential sector is responsible for the largest portion of electricity consumption, accounting for 38.4%. Following closely behind is the industrial sector, which consumes 27.9% of the total electricity usage. The shops and other commercial entities category represent 13.8% of the consumption. Lastly, governmental entities account for 5.5% of electricity consumption. The figure below exhibits the energy consumption percentage across different categories.

Figure 1: Energy consumption % across different segments (percentage are rounded), Source EEHC

While the residential sector remains the largest consumer, its consumption has decreased by 4.4% compared to the consumption levels in 2018/2019 although the number of subscribers has increased from 36.4 million subscribers to 39.1 million subscribers. All other sectors have followed a suit and shown also decrease in consumption. The agriculture sector has seen the greatest decrease, with consumption down by 16.5%. This is followed by governmental entities, which has seen a 15% decrease, and shops and other businesses, which have seen a 13.5% decrease. Industry and utilities have both seen more modest decreases, with consumption down by 2.8% and 7.5% respectively. The next figure shows the electricity increase in each sector.

Figure 2: Development of energy consumption in different sectors: EEHC

## EE targets and plans

Egypt has approved its NEEAP II for the period of 2018-2022, which aims to achieve an ambitious target of 18% energy savings by 2030. In developing this plan, the previous challenges of the NEEAP I, the new energy strategy, and energy law were all taken into account. The NEEAP is also committed to enhancing the institutional structure of EE activities by empowering and completing energy efficiency units across various economic sectors, in line with the Cabinet's 2009 decree for the adoption of an Energy Efficiency Institutional Framework (EEIF) that relies on centralized planning and coordination, as well as decentralized implementation measures.

NEEAP II is committed to promoting EE in both supply and demand. On the supply side, NEEAP has implemented a tariff restructuring program that is expected to result in significant electricity savings. It is essential to have a realistic pricing system for electricity generation, transmission, and distribution costs. NEEAP II estimates that the rationalization of fuel consumption for electricity generation could reach 12.9 million tons of oil equivalent annually. To increase energy efficiency, NEEAP II suggests measures such as reducing distribution network losses, implementing network control, and using smart meters. Furthermore, NEEAP encourages Electricity Distribution Companies (DISCOs) to adopt measures to promote energy efficiency among consumers, as outlined in the Electricity Law.

When it comes to EE on the demand side, there are various measures that can be taken to improve it. These measures are aimed at sectors such as buildings, tourism, industry, public lighting, and education. One particular program focuses on promoting the use of LED lightbulbs in residential areas. This program targets the first three tariff segments, which make up 50% of total consumers in this sector. The cost of LED lightbulbs has been a challenge for these consumers, even though they can lead to savings due to the high contribution of lighting loads in these tariff segments.

The EE target in the NEEAP II is following the integrated sustainable energy strategy target which is achieving 18% of expected energy consumption in 2035. In addition, according to the updated NDC published in 2022, Egypt plans to reduce GHG emissions in the electricity sector by 33% by 2030 compared to the baseline in 2015. Energy efficiency is a key technology to achieve this target. The EE target is to be achieved by:

- implementation of maintenance, upgrade, and replacement programs for outdated power plants. This can involve converting basic gas turbines into combined cycle power plants, installing supercritical steam units, and other similar measures

- activate the role of DISCOs and promote both large and small-scale decentralised renewable energy systems for subscribers. This can be done by implementing Sustainable Energy Action Plans as required by Electricity Law 87/2015

- modernizing the transmission and distribution networks, including Extra High voltage substations, control centres, and smart grids. The shift towards smart grids will substantially improve the effectiveness of electricity distribution, lower the cost of infrastructure investments for electrical networks, and reduce carbon emissions

Moreover, Egypt indicates some energy efficiency and energy conservation measures to be taken in building sectors in updated NDC and include:

- The promotion of RE and EE in both new and existing establishments is crucial. It is important to prioritize sustainability interventions in accordance with national priorities. This involves the installation of rooftop PV panels for electricity generation, implementation of 5,300 solar water heaters, and an expansion in the use of LED lighting in the residential sector by 2030

- Aiming to enhance EE labeling and appliance specification programs, eliminate non-energy efficient equipment, and educate consumers about the benefits of purchasing alternative energy-efficient home appliances

- One way to promote green buildings is by enforcing EE codes for new constructions and implementing procedures for renovating existing structures to meet energy performance standards (EPS). Additionally, voluntary guidelines for sustainable building practices can be adopted, and incentives can be introduced to encourage the use of the best available technologies. The Housing and Building Research Center (HBRC) has already developed Energy Efficiency Building Codes for new residential, commercial, and government buildings, which were enacted through ministerial decrees in 2005, 2009, and 2010. By 2030, the plan is to develop 16,960 residential units that adhere to green building standards and increase awareness and community involvement in sustainable building practices

## EE standards and regulations

Egypt has several laws, regulations, and policies that are related to EE. The below table summarizes the laws that are related to EE.

Table 5, Laws that are related to EE

| LAW | STATUS | YEAR |
| --- | --- | --- |
| Decree No. 239 for the year 2018 issuing the executive regulations of the law for Gas Market activities regulation Act No. (196) of the year 2017 | in force | 2018 |
| Law No. 196 of 2017 for Gas Market Activities Regulation | In force | 2017 |
| Executive regulation for new electricity Law | In force | 2016 |
| New Electricity Law 87 for the year 2015 | In force | 2015 |

The Gas Regulatory Authority (GasREG) is granted extensive powers by the Law No. 196/2017 and its executive regulations to fulfill its objectives. These powers include establishing development and business plans, work programs, management rules, and techniques. GasREG is also responsible for ensuring that licenses issued comply with efficient energy resource utilization, health, safety, and environmental laws and regulations, gas market laws, and regulations. Additionally, GasREG may request any necessary information or data in connection with the license.

The new electricity law 87/2015 2015 represents the legal framework dedicated to EE promotion and stated the institutional framework for the national entity in charge of EE policies design and development as shown in Figure 3.

Figure 3: Institutional framework for EE

The EE industry is regulated by Electricity Law No. 87 of 2015. According to the law, the Council of Ministers will appoint a body to collaborate with relevant entities to implement EE policies. The law aims to achieve the following objectives:

- Apply EE specifications and labels to electrical equipment and devices used by consumers

- Replace non-energy efficient devices and equipment in accordance with the Executive Regulations of the Law

- Enhance EE in industrial and commercial systems

- Establishments with contractual capacities exceeding 500 kW must appoint an energy manager to oversee energy efficiency measures/energy conservation measures (EEMs/ECMs) and maintain an energy register, according to the law

- The Executive Regulations will regulate this requirement, and the energy register will be supervised and monitored by the body designated by the Cabinet

The Executive Regulations of the Electricity Law provide a detailed outline of the supervisory body's responsibilities, in conjunction with relevant ministries, authorities, and entities, to develop necessary policies and regulations for replacing non-energy efficient devices. The key responsibilities include:

- Establishing the Terms of References (ToRs) for granting licenses to laboratories to conduct the required tests for verifying the EE of electrical equipment and devices

- Conducting studies on expanding the application of EE specifications and labels on equipment and appliances

- Proposing incentives to encourage the replacement of appliances

- Evaluating Energy Efficiency Improvement Projects (EEIPs) in industrial and commercial establishments, as well as in public and administrative buildings

- Evaluating programs focused on replacing non-energy efficient appliances

- Conducting studies on collaborating with the Ministry of Finance and financial organizations to establish funds for securing the required funding for replacing non-energy efficient appliances within the framework of improving EE

### Building standards and regulation

The building sector in Egypt is highly energy-intensive, accounting for 50.5% of the country's total final energy consumption. Therefore, it is crucial to implement EEMs/ECMs in this sector. These measures may involve applying insulation standards, developing EE building codes, and establishing EE labelling systems for buildings.

The NEEAP I for EE in buildings is divided into three phases: short term (2015-2020), medium-term (2020-2024), and long term (2025-2035). The short-term plan aims to increase awareness among energy users about equipment energy labels and to review the EE code in buildings. The goal is to take executive actions to implement the building code and prescribe obligatory minimum level of energy performance for new buildings as well as in case of building renovation.

Egypt has enacted Building Energy Efficiency Codes (EEBCs) that enforce mandatory energy performance standards for residential, commercial, and public buildings through three code documents. However, these codes are currently not being enforced. A ministerial decree (401/1987) was also issued by the Minister of Housing mandating the installation of solar water heating systems in new governmental buildings in new communities. Unfortunately, this decree has also yet to be enforced.

Implementing EEBCs is crucial to ensure EE standards are met during the design, construction, and renovation of residential and commercial buildings. This includes setting minimum standards for the building's envelope, systems, and equipment. The process of establishing and implementing EEBC requirements involves several stages, which are summarized in Table 6.

Table 6: EEBCs areas

| EEBC | STATUS |
| --- | --- |
| Technical Requirements | Existing |
| Legal Framework | Existing |
| Institutional setup (clarity regarding responsibilities) | Planned |
| Implementation | Does not exist |
| Enforcement procedures | Does not exist |
| Monitoring & Evaluation procedures | Does not exist |

Building codes for EE can be either prescriptive or performance-based. Prescriptive codes have fixed EE requirements for specific building elements and equipment, which makes them easier to implement and verify. However, they lack flexibility and need frequent updates to keep up with changing technology. On the other hand, performance-based models focus on the overall energy performance of the building, taking into account energy consumption, fossil fuel usage, and greenhouse gas emissions. These models are more flexible and adaptable to new technologies but require higher engineering skills due to their complexity. In Egyptian EEBCs, both types either prescriptive and performance-based exists.

### Appliances standard and regulations

Energy usage by appliances in residential areas is a major concern due to their high consumption. With a rapidly growing population and urbanization, it's imperative that we prioritize energy-efficient measures for appliances. One way to achieve this is through the implementation of EE standards and labels, which promote lower energy requirements for appliances. This can be done by either phasing out inefficient models or providing transparent information to users, who are likely to choose more efficient options. The two most significant policies for appliances are Minimum Energy Performance Standards (MEPS) and Energy Labeling.

MEPS serve as a crucial policy tool in promoting EE for home appliances. This is achieved by prohibiting the sale of the least efficient appliances in the market, prompting manufacturers to innovate and encouraging consumers to adopt more energy-efficient options.

The second policy is the Labelling policy. Appliances are often labeled with energy-efficiency labels that provide information on the product's energy performance, including energy use, efficiency, and cost. These labels are designed to help consumers make informed purchasing decisions. There are two common types of labels: endorsement labels and comparative labels.

Energy labels can either be used on their own or alongside energy standards. They not only inform consumers who prioritize EE about the products they purchase, but also establish a common benchmark for EE that utility companies and government agencies can use to incentivize the purchase of energy-efficient products. The effectiveness of energy labels is contingent on how well they convey information to consumers, as well as the support they receive from information campaigns, financial incentives, and other related programs.

The following table exhibits the appliances that are covered by MEPS and labeling policy in Egypt.

Table 7, Appliances covered by MEPS and labelling policies

| APPLIANCE | POLICY IMPLEMENTED |
| --- | --- |
| Air condition | MEPS, Labelling |
| Lighting | MEPS, Labelling |
| Refrigerator/ Freezer | MEPS, Labelling |
| Washing machines | MEPS, Labelling |
| Laundry Dryer | MEPS, Labelling |
| TV | MEPS, Labelling |
| Vacuum cleaners | MEPS, Labelling |
| Fans | MEPS, Labelling |
| Water electric heaters | MEPS, Labelling |
| Electric Ovens | MEPS, Labelling |
| Dishwasher | MEPS, Labelling |

## Key programs

The following table exhibits some of implemented EE programs in Egypt.

Table 8: Implemented EE programs in Egypt

| PROGRAM | DURATION | FUNDING SOURCE | IMPL ORG. | OBJECTIVES |
| --- | --- | --- | --- | --- |
| Egyptian-German Energy Partnership (EGEP) | 2012-ongoing | German Government, European Union | MoERE | To promote EE, RE, and energy security in Egypt through various initiatives, including the development of EE standards for buildings and the establishment of a national EE Fund |
| Sustainable Energy Finance Program (SEF) | 2013-2018 | European Union, Egyptian Government | MoIC | To support the financing of EE and RE projects in Egypt by providing TA to FIs to help them develop sustainable energy finance products and services |
| EE and RE Program (EE-REP) | 2015-2020 | United States Agency for International Development (USAID) | MoERE | To promote EE and RE in Egypt through TA and grants to support the implementation of EE and RE projects in various sectors, including buildings, industry, and transportation |
| Green Homes program | 2019-ongoing | Ministry of Housing, Utilities, and Urban Communities | MoERE | To provide financing and technical assistance to support the development of energy-efficient and environmentally sustainable housing projects in Egypt |
| Industrial Energy Efficiency Fund (IEEF) | 2018-ongoing | Global Environment Facility (GEF) | RCREEE, UNIDO | The IEEF is a financial instrument that supports the delivery of technical services for the industrial energy efficiency in Egypt through an Energy Management Assistance (EnMA) Service |

# Market demand assessment

Market demand assessment covers the three target sectors: residential, commercial and public sectors. Due to data shortage, residential sector will be assessed as one segment. Some segments under commercial and public will be combined as there are the same type of buildings. For example, public hospitals are classified under public, while private hospitals are classified under private sector. Due to the absence of data, private hospitals and public hospitals will be treated as one segment and discussed under commercial sectors.

## Residential sector

### Sector overview

Residential sector is the sector with highest share of electricity consumption. According to latest EEHC report 2021/2022, residential sector consumed 38.4% of the total electricity consumption in Egypt. While the consumption of the residential sector is increasing, its share in the overall consumption is slightly decreasing in the last two years.

Figure 4: Development of residential sector energy consumption. Source: EEHC, 2023

In Egypt, the majority of buildings belong to the residential sector. The total floor area for residential buildings is estimated to be 2.925 billion square meters, with 85% (2.485 billion square meters) being designated for multifamily housing and the remainder for single family homes. Residential sector new construction has an annual growth rate between 2 and 2.5%.

In the residential sector, there are a total of 33,623,000 subscribers who can be categorized based on their consumption segment. There are seven segments in total. 62.4% of the subscribers fall into the lowest three consumption segments, which are below 200 kWh.11.5% of the subscribers fall into the monthly consumption segment below 50 kWh, while 14.8% and 36% fall into the consumption segments of 50-100 kWh and 0-200 kWh, respectively. The lowest consumption segment (below 50kWh) includes closed and unoccupied apartments. The next figures exhibit the percentage of subscriber in each consumption categories.

Figure 5: Consumption categories distribution

### Technical assessment

The total residential energy consumption in 2021/2022 was 62,912GWh. By dividing this value over the total floor area of the residential sector, the specific electricity consumption will be 21.51 kWh/m2. However, this does not show the complete picture. Egypt has eight different climatic areas according to the Housing and Building Research Centre (HBRC). This will imply different pattern of energy consumption in each area. The next Figure 6 shows the different climatic areas in Egypt.

Figure 6: Climatic areas in Egypt, Source: Ali. K Abdel-Rahman et al

In a study published in 2013, almost 1,500 apartments in Egypt were surveyed in three cities to identify energy consumption patterns. In addition, the study classified the apartments into two typologies. The below table shows the specific energy consumption that were identified from the survey. The table shows numbers comparable to what have been calculated above (21.51 kWh/m2). Moreover, it proves there is different value for specific electricity consumption according to the location and the typology of the apartment.

Table 9: Specific electricity consumption in different cities in Egypt

| AVERAGE ANNUAL SPECIFIC CONSUMPTION | CAIRO kWh/m2 | ALEXANDRIA kWh/m2 | ASYUT kWh/m2 |
| --- | --- | --- | --- |
| Typology 1 | 26.6 | 22.4 | 31 |
| Typology 2 | 14 | 11 | 18 |

The team tried to collect recent data for the pattern of energy consumption in residential sector in Egypt. The most comprehensive study found was done by the World bank and published in 2017. Moreover, during the interviews, several stakeholders pointed out to this study as their source of data.

The World bank study indicated that on aggregated level, lighting is the largest application the consumes electricity and represent 31%, followed by air condition (AC) and refrigeration and water heating (WH) with percentage of 13%, 13%, and 11% respectively.

As the Egyptian government has implemented a program to distribute efficient LED bulbs to its residents. The share of lighting in total residential consumption may have decreased over the years, there is currently no recent statistics or studies available to confirm this.

According to the World bank study, residential sector has a saving potential between 30% to 50%, this could be translated to savings of 18,874 GWh to 31,456 GWh. This represents from 11.5% to 19.2% of the total electricity consumption in Egypt. Regarding carbon emission savings it will be estimated to be between eight million tons and 13 million tons of equivalent carbon emissions.

Figure 7: Residential electricity consumption categories, Source: The world bank 2017

Moving from aggregated level to individual level to determine the most significant electricity consumers. According to study carried out by Japan International Cooperation Agency (JICA), AC and refrigerators are the most significant consumers. JICA study installed power monitoring devices on each individual appliances in five houses in Alexandria for a period of one year. The next table shows the results of the study.

Table 10, JICA Study results

| House | AC | Refrigerator | Light | WH | TV | OTHERS |
| --- | --- | --- | --- | --- | --- | --- |
| House 1 | 43.30% | 14.70% | 17.80% | 17.40% | 4.60% | 2.2% |
| House 2 | 19.80% | 35.60% | 18.20% | 18.70% | 5% | 7.7% |
| House 3 |  | 50.00% | 17.70% | 14.20% | 6.80% | 18.1% |
| House 4 |  | 52.60% | 22.70% | 12.90% | 4.60% | 11.8% |
| House 5 |  | 38.00% | 15.80% | 35.90% | 6.90% | 10.3% |

*	Only house# 1 & 2 have ACs, House #3 has two refrigerators and two Freezers, House #4 has two refrigerators, House #5 has two water heaters

It could be concluded that, AC, refrigerators, water heaters, and lighting are the most significant electricity consumers in the residential sectors. The below figure shows that specific electricity consumption for each house is in the normal range calculated before. House #3 has the largest consumption due to the existence of two refrigerators and two freezers (50% of the total consumption) that are working 24/7. House three has two water heaters consuming 35.9% of the total consumption.

Figure 8: Specific electricity consumption

As explained in the above chapter, there are seven consumption categories. The EE potential, as well as the higher purchasing power, makes the highest four consumption categories as primary targets of the EE Fund. To estimate and match the number of subscribers with the total energy consumption the following assumptions were made:

- The households (HH) in the lowest consumption category are not occupied

- The HHs in the second lowest consumption category are partially occupied

- The average monthly energy consumption for each consumption category is on the lower threshold

The below table shows the assumed no of subscribers, average monthly consumption, and total annual consumption of each consumption category.

Table 10: xxx

| CONSUMPTION CATEGORY | ASSUMED AVERAGE CONSUMPTION, (monthly, in kWh) | ASSUMED no OF SUBSCRIBERS | TOTAL CONSUMPTION (monthly, in kWh) | TOTAL CONSUMPTION (annual, in GWh) |
| --- | --- | --- | --- | --- |
| 0-50 | 0 | 0 | 0 | 0 |
| 51-100 | 75 | 3,253,472 | 244,010,400 | 2,928 |
| 0-200 | 110 | 12,104,280 | 1,331,470,800 | 15,978 |
| 201-350 | 205 | 8,583,952 | 1,759,710,160 | 21,117 |
| 351-650 | 400 | 3,241,257 | 1,296,502,800 | 15,558 |
| 0-1000 | 660 | 558,142 | 368,373,720 | 4,420 |
| more than 1000 | 1001 | 258,897 | 259,155,897 | 3,110 |

The energy consumption of the last four categories equalling 44,205 GWh is used as the baseline for the calculation of the energy saving potential. Considering the World Bank assumption of energy saving potential between 30 and 50%, the savings for the target group range from 13,261 GWh to 22,102 GWh.

### Energy Conservation measures (ECMs)

#### Air Conditioning

In Egypt, local manufacturers dominate the AC market, with most international brands having local partners who manufacture their technology, especially for small AC equipment like split systems. The most commonly installed AC system type in the current building stock, particularly in the residential sector, is the single split system, with a majority of them being ductless. The EER W/W of AC systems installed in existing buildings ranges between 2.7-3.7, with the majority having an efficiency of 2.9-3.2. This is significantly lower than the best available efficiency range. The minimum energy performance requirement was previously class C at an EER of 3.26, but as of June 2020, it has been changed to class B at a value of 3.51. Starting from June 2022 onwards, it will be 3.81. Below is a table that summarizes the efficiencies of currently installed, new equipment, and available AC systems.

Table 12: Different AC efficiencies

| SYSTEM TYPE | AVERAGE EFFICIENCY IN THE STOCK | EFFICIENCY INSTALLED IN NEW BUILDINGS | BEST AVAILABLE EFFICIENCY |  |
| --- | --- | --- | --- | --- |
|  |  |  | National | International |
| Window type | 2.8 - 3 | NA | - | - |
| Single Split System | 3.0 | 2.5-05.3 | 5.38 | 6.5 |
| VRFs/Multi Split | 3.5 - 4.5 | 3.5 - 4.5 | 4.5 | 5 - 4.4 |

According to estimates, there are approximately 5.3 million AC systems and 0.28 million chillers currently installed in buildings in Egypt. In 2018, the AC market in Egypt had a value of EUR 280 million, with 520,000 units sold. Another study suggests that the sales figures were slightly higher in 2017, with around 580,000 units sold, of which 92% were locally assembled. This is due to the high custom duty imposed on imported products, which accounts for approximately 40% of the product value, encouraging local manufacturing.

Single split systems make up over 90% of the market in terms of units sold, with ductless systems representing 85% of that share. Fan coil systems and window or wall AC units make up 5% and 3%, respectively. The remaining 2% of the market includes AHUs, rooftop systems, chillers, and VRF systems. Figure 9 displays the breakdown of these shares based on the number of systems sold.

Figure 9: Shares of different types on cooling appliances

For the market assessment the assumption is to replace 500,000 non-inverter AC units with inverter AC units over a period of 5 years. The average annual consumption of AC is 1,500 kWh. The cost for non-inverter 2.25 HP AC and inverter AC are EGP 21,750 and EGP 27,085, respectively. With an estimation of 50% expected savings over the implementation period are 375 GWh. With estimation that the average price difference is EGP 5,500 between non-inverter AC and inverter AC, the next table summarize the calculation of the savings and payback period according to each consumption categories

Table 13: Savings per consumption category

| CONSUMPTION CATEGORY | 201-350 | 351-650 | 0-1000 | more than 1000 |
| --- | --- | --- | --- | --- |
| Tariff | 1.23 | 1.36 | 1.36 | 1.45 |
| Replaced units | 25,000 | 25,000 | 25,000 | 25,000 |
| Annual savings (kWh) | 18,750,000 | 18,750,000 | 18,750,000 | 18,750,000 |
| Annual savings (EGP) | 23,062,500 | 25,500,000 | 25,500,000 | 27,187,500 |
| Annual cost (EGP) | 125,000,000 | 125,000,000 | 125,000,000 | 125,000,000 |
| Payback period (years) | 5.96 | 5.39 | 5.39 | 5.06 |

As indicated before, 500,000 AC units will be replaced over 5 years period., the total amount of required funding is EGP 2.75 billion or EUR 80.8 million. The total emission savings over the implementation period is 478,125 tons CO2.The next table summarize the financing needs according to each implementation year, annual emission savings and annual primary energy savings.

Table 14: Annual financing needs

|  | YEAR 1 | YEAR 2 | YEAR 3 | YEAR 4 | YEAR 5 | TOTAL |
| --- | --- | --- | --- | --- | --- | --- |
| Financing needs p.a. (EGP) | 550,000,000 | 550,000,000 | 550,000,000 | 550,000,000 | 550,000,000 | 2,750,000,000 |
| Financing needs p.a. (EUR) | 16,176,471 | 16,176,471 | 16,176,471 | 16,176,471 | 16,176,471 | 80,882,353 |
| Emission savings p.a. (kg CO2) | 31,875,000 | 63,750,000 | 95,625,000 | 127,500,000 | 159,375,000 | 478,125,000 |
| Primary energy savings p.a. (kWh) | 192,307,692 | 384,615,385 | 576,923,077 | 769,230,769 | 1,153,846,154 | 3,076,923,077 |

#### Lighting

Lighting is one of the applications that has significant electricity consumption in residential sector. Currently there is no incandescent bulbs in the market as they are banned to be imported or manufactured. However, there is still manufacturing activities for florescent bulbs. Replacing this type of lamp with LED lamps can save considerable amount of electricity.

Our assumption here is that each household in the selected consumption category will replace 3 bulbs each year for a period of 5 years. The average savings per bulb is 10 W and the average cost difference is EGP 58 as currently offered from one of the lamp manufacturers in Egypt. The average annual working hours are estimated with 2,920. The below table summarizes the annual savings and costs per consumption category.

Table 15: Savings per consumption category

| CONSUMPTION CATEGORY | 201-350 | 351-650 | 0-1000 | more than 1000 |
| --- | --- | --- | --- | --- |
| Tariff (EGP) | 1.23 | 1.36 | 1.36 | 1.45 |
| Replaced units | 25,751,856 | 9,723,771 | 1,674,426 | 776,691 |
| Annual savings (kWh) | 751,954,195 | 283,934,113 | 48,893,239 | 22,679,377 |
| Annual savings (EGP) | 924,903,660 | 386,150,394 | 66,494,805 | 32,885,097 |
| Annual cost (EGP) | 1,493,607,648 | 563,978,718 | 97,116,708 | 45,048,078 |
| Payback period (years) | 1.61 | 1.46 | 1.46 | 1.37 |

The total amount of required funding is EGP 2.1 billion or EUR 323.4 million. The next table summarizes the financing needs according to each implementation year, annual emission savings and annual primary energy savings.

Table 16: Annual Financing needs

|  | YEAR 1 | YEAR 2 | YEAR 3 | YEAR 4 | YEAR 5 | TOTAL |
| --- | --- | --- | --- | --- | --- | --- |
| Financing needs p.a. (EGP) | 2,199,751,152 | 2,199,751,152 | 2,199,751,152 | 2,199,751,152 | 2,199,751,152 | 2,199,751,152 |
| Financing needs p.a. (EUR) | 64,698,563 | 64,698,563 | 64,698,563 | 64,698,563 | 64,698,563 | 323,492,816 |
| Emission savings p.a. (kg CO2) | 470,670,893 | 941,341,786 | 1,412,012,679 | 1,882,683,572 | 2,353,354,465 | 7,060,063,396 |
| Primary energy savings p.a. (kWh) | 2,839,643,397 | 5,679,286,794 | 8,518,930,191 | 11,358,573,588 | 14,198,216,985 | 42,594,650,954 |

#### Refrigerators

Unlike AC, there is no data on how many refrigerators are sold annually in Egypt. However, according to Central Agency for Public Mobilization and Statistics (CAPMAS), 6.7 million residential refrigerators were produced in 2018. This production is for local and export markets.

The market already offers local manufactured refrigerators with A class efficiency which is the most common and the lowest class allowed for sale in the market. However, for higher classification (e.g., A+, A++, A+++), the refrigerators are imported. Our comparison here will compare A class with A+++ refrigerators.

The annual energy consumption of according to EE label- locally 355 litre refrigerators with A class efficiency label is 696 kWh. The estimated price is EGP 13,000. While an imported refrigerator with a capacity of 310 litre and has an efficiency class of A+++ has an annual energy consumption of 248 kWh. The price is EGP 29,000. The annual electricity savings will be 448kWh.

NEEAP II estimated that replacing 1 million refrigerators. The assessment will follow this estimation of replacing 1 million refrigerators and the estimated implementation period is 5 years. The following table exhibits the annual savings, annual cost -with average financing needs to one refrigerator is EGP 20,000 to cover the cost gap between class A and class A+++ refrigerators and payback period.

Table 17: Savings per consumption category

| CONSUMPTION CATEGORY | 201-350 | 351-650 | 0-1000 | more than 1000 |
| --- | --- | --- | --- | --- |
| Tariff | 1.23 | 1.36 | 1.36 | 1.45 |
| Replaced units | 50,000 | 50,000 | 50,000 | 50,000 |
| Annual savings (kWh) | 22,400,000 | 22,400,000 | 22,400,000 | 22,400,000 |
| Annual savings (EGP) | 27,552,000 | 30,464,000 | 30,464,000 | 32,480,000 |
| Annual cost (EGP) | 1,000,000,000 | 1,000,000,000 | 1,000,000,000 | 1,000,000,000 |
| Payback period (years) | 36.30 | 32.83 | 32.83 | 30.79 |

Two reasons could explain long payback period. First one is the low electricity price. The government has set a plan to remove subsidies and achieve cost reflective electricity tariffs. However, this plan was prepared in 2020, when the exchange rate was EGP 16 to one US Dollar. Currently, the exchange rate doubled and equals EGP 31. Electricity tariff did not change to reflect the incremental increase in fuel and spare parts cost. The second reason is that the suppliers calculated the cost of the appliances based on black market exchange rate which is 30% higher than official exchange rate.

The total investment needs are EGP 20 billion or EUR 588.2 million. The next table shows the annual investment needs, emission savings and primary energy savings.

Table 18: Annual financing needs

|  | YEAR 1 | YEAR 2 | YEAR 3 | YEAR 4 | YEAR 5 | TOTAL |
| --- | --- | --- | --- | --- | --- | --- |
| Financing needs p.a. (EGP) | 4,000,000,000 | 4,000,000,000 | 4,000,000,000 | 4,000,000,000 | 4,000,000,000 | 20,000,000,000 |
| Financing needs p.a. (EUR) | 117,647,059 | 117,647,059 | 117,647,059 | 117,647,059 | 117,647,059 | 588,235,294 |
| Emission savings p.a. (kg CO2) | 38,080,000 | 76,160,000 | 114,240,000 | 152,320,000 | 190,400,000 | 571,200,000 |
| Primary energy savings p.a. (kWh) | 229,743,590 | 459,487,179 | 689,230,769 | 918,974,359 | 1,148,717,949 | 3,446,153,846 |

#### Solar Water Heater (SWH)

According to CAPMAS, 3.8 million electric heaters were manufactured in 2018. Electric heaters consume 11% of the total electricity consumption in residential sector leading to annual consumption of 6,920 GWh. NEEAP II suggested installing 5,000 solar water heaters instead of electric water heater. The estimated electricity savings over a three-year implementation period are 25.1 GWh.

The 2 m2 solar water heater will save 1,674 kWh of electricity. The estimated cost is for the solar water heater is EGP 30,000. For the market assessment, the assumption is to replace 8,000 electric water heaters with solar water heaters over a period of five years. The below table summarizes the annual savings and payback period according to each consumption categories. The reasons for the long payback periods are the same as explained in the above section.

Table 19: Savings per consumption category

| CONSUMPTION CATEGORY | 201-350 | 351-650 | 0-1000 | more than 1000 |
| --- | --- | --- | --- | --- |
| Tariff | 1.23 | 1.36 | 1.36 | 1.45 |
| Replaced units | 400 | 400 | 400 | 400 |
| Annual savings (kWh) | 669,600 | 669,600 | 669,600 | 669,600 |
| Annual savings (EGP) | 823,608 | 910,656 | 910,656 | 970,920 |
| Annual cost (EGP) | 12,000,000 | 12,000,000 | 12,000,000 | 12,000,000 |
| Payback period (years) | 14.57 | 13.18 | 13.18 | 12.36 |

The total financing needs over the implementation period is EGP240 million or ca. EUR 7 million. The below table shows the annual financing needs, annual emission savings and annual primary energy savings.

Table 20: Annual financing needs

|  | YEAR 1 | YEAR 2 | YEAR 3 | YEAR 4 | YEAR 5 | TOTAL |
| --- | --- | --- | --- | --- | --- | --- |
| Financing needs p.a. (EGP) | 48,000,000 | 48,000,000 | 48,000,000 | 48,000,000 | 48,000,000 | 240,000,000 |
| Financing needs p.a. (EUR) | 1,411,765 | 1,411,765 | 1,411,765 | 1,411,765 | 1,411,765 | 7,058,824 |
| Emission savings p.a. (kg CO2) | 1,138,320 | 2,276,640 | 3,414,960 | 4,553,280 | 5,691,600 | 17,074,800 |
| Primary energy savings p.a. (kWh) | 6,867,692 | 13,735,385 | 20,603,077 | 27,470,769 | 34,338,462 | 103,015,385 |

### Summary

The table below summarizes the expected energy savings, financial savings, emission savings, CAPEX, primary energy savings and payback period over the implementation period of 5 years.

Table 21: xxx

| APPLIANCES | Energy savings GWh p.a. | Financial savings EUR/p.a. | Emissions savings tons CO2E p.a. | Primary energy savings GWh p.a. | Required financing (EUR p.a.) | Average SPP (years) |
| --- | --- | --- | --- | --- | --- | --- |
| LED replacement | 1,107 | 41,483,352 | 470,671 | 2,840 | 64,698,563 | 1.56 |
| Replacing non-inverter AC with inverter AC | 75 | 2,977,941 | 31,875 | 192 | 16,176,471 | 5.43 |
| Replacing 1 million refrigerators | 89.6 | 3,557,647 | 38,080 | 230 | 117,647,059 | 33.07 |
| Installing 8,000 SWHs | 2.6 | 106,348 | 1,138 | 7 | 1,411,765 | 13.27 |
| Total | 1,275 | 48,125,288 | 541,764 | 3,269 | 199,933,857 |  |
| Total over implementation period (5 years) | 19,196 | 721,879,322 | 8,126,463 | 49,221 | 999,669,287 |  |

### Estimated market size

The estimated market potential in the residential sector for the above selected appliances is approximately EUR billion. However, the EE Fund will not be able to target 100% of the market. The below table exhibits the different estimated market shares from 1% to 5%.

Table 22: xxx

| MARKET SHARE (%) | MARKET SHARE (EUR) |
| --- | --- |
| 1% | 9,996,693 |
| 2% | 19,993,386 |
| 3% | 29,990,079 |
| 4% | 39,986,771 |
| 5% | 49,983,464 |

### Case study

The case study presented below is a follow up of the JICA study that was included in Section 4.1.3.1. After measuring the electricity consumption in one of the apartments, the AC electricity consumption percentage was 43.3% of the total electricity consumption of the house. Replacing The current AC with inverter AC reduced the electricity consumption considerably. The next Figure 10 and Figure 11 shows the electricity consumption before and after replacing the AC. The simple payback period was 3.9 years.

| Figure 10: Electricity consumption before replacing AC | Figure 11: Electricity consumption after replacing AC |
| --- | --- |

### Conclusion

The residential sector is the sector with the highest electricity consumption in Egypt. Replacement of lighting technology is the measure with the highest potential. However, due to the ban of inefficient incandescent lightbulbs and previous initiatives for the introduction of LED, the potential might be lower than anticipated. Replacement of non-inverter to inverter AC is the measure with the second highest potential for savings followed by refrigeration, and water heating. It must be noted that electricity consumption patterns change according to location, space, number of persons, and number of appliances.

LED bulbs have currently high market penetration, there are fluorescent bulbs that are manufactured and sold in the market. LED bulbs is still more expensive however the payback period is less than 2 years.

Inverter AC is the most promising appliance, as it is manufactured locally, has short payback period, and could save considerable amount of energy and CO2 emissions. Replacing refrigerator with more efficient one will have a long payback period due to low electricity price and high currency exchange rate. Replacing electric heater with Solar water heater would has a payback period higher than replacing ACs and shorter than replacing to more efficient refrigerators.

Implementing EE programs over proposed five years would lead to saving of 6,374 GWh over implementation period. Although some measures have long payback periods, the average payback period for all implemented measures is less than 5 years. Total emissions saving is 2.7 million tons of equivalent CO2.

With 1% market capture the financing needs is approximately EUR 10 million.

## Commercial sector

### Sector overview

Commercial sector consumed 13.8% of the total electricity consumption in Egypt in the fiscal year 2021/2022. The sector electricity consumption reached 22,720 GWh. In addition, the sector is one of the sectors that has significant growth rate. The sector has increased by 15.6% since fiscal year 2017/2018. The commercial sector has the second largest number of subscribers after the residential sector.

The commercial sector has 802,231 buildings representing 5.4% of the total building stock in Egypt. Commercial building savings potential ranges between 40-60%. Cooling, ventilation, and Air Conditioning (HVAC) is the applications with the highest potential to reduce electricity consumption.

In this study, the commercial sector will include several segments such as office buildings, resorts & hotels, restaurants, schools, hospitals. The study team faced difficulties while collecting data. The most relevant data that was found from 2015/2016 in a report that was published by EgyptERA and CAPMAS. The report indicates the electricity consumption percentage of some segments in relation to the total electricity consumption. As data is rather outdated, KfW provided a recent estimation for commercial sector segments (Table 23).

Table 23: Electricity consumption for the segments of the Commercial sector

| COMMERCIAL SECTOR - SEGMENTS | GWh |
| --- | --- |
| Office buildings | 133 |
| Hotels & resorts | 593 |
| Shopping malls | 267 |
| Stores | 391 |
| Private schools | 138 |
| Private universities | 279 |
| Priv. Hospitals & medical centers | 424 |
| Restaurants | 170 |
| Private kindergartens | 8 |
| Others | 3,430 |
| Total Commercial (GWh) | 5,833 |

During the study preparation EgyptERA provided us with data from some DISCOs. Both data – KfW data and DISCOs data- were compared and acceptable discrepancies were found. The team decided to use the data provided from KfW for the market assessment

The approach to use that data is as following:

For the demand assessment in commercial sector, the following approach is used:

- Brief overview per segment based on the available data explaining the energy consumption and most significant energy users

- Introduce ECMs the most significant users and the savings potential

- Exhibit a summary of results per segment

### Technical assessment

#### Hotels

##### Energy consumption profile

According to CAPMAS, in 2019, Egypt has 1,335 hotels & resorts, total of 163,316 rooms and 307,303 beds. In the same year, the total guests were 13.026 million guests, and the total registered nights are 135.373 million. This will lead an average of 10.4 nights per guest.

There is shortage in studies and audits that are published or available to estimate the savings potential for hotels in Egypt. However, there is an EBRD study prepared in 2013 that could provide some estimations. The study surveyed 93 hotels and carried out 11 walkthrough audits in red sea and Sinai area that contain nearly 60% of all hotels and resorts in Egypt according to the study. The savings percentages were in the range of 18% (the minimum) to 40% (the maximum). In a recent walkthrough audit that was done by one of the GFA consulting team in a hotel in Cairo, although the hotel was following the best practices and investing consciously in EE, a 13% savings of the total energy consumed could be achieved. With a conservation assumption of 20% savings, the overall potential savings in hotels is 119 GWh.

Hotels have different sources of energy, especially hotels that are not connected to the electricity grid and depend on diesel to provide all or part of electricity to the buildings. On average and according to the study, electricity has the lion’s share in energy cost and represent around 60% of the total energy cost.

Electricity used in cooling consumes on average 45% of the total electricity used, followed by lighting (15%), motors and pumps (13.75%), kitchen (11.25%) and others (15%). The following figure shows the electricity consumption breakdown from four representative hotels.

Add figure with the shares as described in the paragraph above and amend name of the Figure – not clear

Figure 12: Electricity consumption breakdown

Using the above breakdown, the electricity consumption in hotels could be estimated in the following table.

Table 24: xxx

| MEASURES | PERCENTAGE | ANNUAL CONSUMPTION (GWh) |
| --- | --- | --- |
| Cooling | 45% | 266.85 |
| Lighting | 15% | 88.95 |
| Motors and pumps | 14% | 81.54 |
| Kitchen | 11% | 66.71 |
| Others | 15% | 88.95 |

When it comes to benchmarking, the KPI for the electricity selected is the consumption per guest/night (PAX). In terms of international benchmarking, the average electrical consumption per guest night in Europe is 55.5 kWh/PAX, New Zealand 43.1 kWh/PAX), Cyprus 24.2 kWh/PAX, and Majorca 14.2 kWh/PAX. It could be seen the diversity in the benchmarking because of different climatic zones, length of high seasons, etc.

In Egypt, when using the number of guest nights in 2019 and the electricity consumption in 2019, the average consumption will be 25.5 kWh/PAX. Nevertheless, the actual consumption could be higher especially in hotels & resorts that are operating around the year. The next figure shows the results from the surveyed hotels.

Add figure and amend name of the Figure – not clear

Figure 13: Electricity consumption for surveyed hotels

The electricity tariff in hotels will be assumed to be the same electricity tariffs for medium voltage consumers. This assumption due the fact that most of hotels are contracted on medium voltage (MV) and install their own transformers to deuce the voltage to low voltage (LV) to reduce expenditures. The tariff for MV in the year 2024/2025 – last year in the reforming plan- is EGP 1.15/kWh and EGP 60/kW as demand fees. The blended rate that will be used is EGP 1.3/kWh.

##### Energy conservation measures ECMs

Several ECMs could be applied to hotels. Based on the literature and the audits carried out before in the eleven hotels, five ECMs have been identified as ECMs to be implemented in hotels and they are summarized below.

###### Upgrade HVAC systems

Typically, hotels have large cooling facilities to provide cool air to common areas and guest rooms. However, some hotels also use standard split air conditioning units to cool guest rooms. For hotels that use individual split units, there is an opportunity to switch to centralized cooling systems when at the end of lifetime. This is due to the higher COP ratios and lower equivalent full load hours of centralized equipment compared to individual units. Additionally, some hotels use air cooled central systems, while water cooled systems are more efficient. The proposed solution is to eliminate the use of AC split units or replace air cooled central AC with water cooled central AC.

###### Building renovation

Improving the EE of an existing hotel by upgrading the insulation of its roof and facades is a challenging task. This measure requires a significant investment and may not be as profitable as other EEMs/ECMs, such as renovating heating or cooling units. Furthermore, the hotel's business may be impacted as rooms undergoing renovation will need to remain closed for a certain period. This measure is not very popular and there is a lack of data on cost estimates in the Egyptian context. Our proposal for improving EE is to replace single glazed windows with double glazed windows, which nearly doubles the insulation.

###### Building Management system

A Building/Energy Management System (BMS/EMS) is a technology-based system that automatically monitors and controls various hotel services, such as air conditioning, ventilation, heating, lighting, and other energy-consuming systems within a building or even multiple buildings. Some BMSs also manage gas and water usage. The suggestion is to implement BMS to regulate HVAC systems, lighting, pumps, and other similar systems.

###### Inverters for pumps and motors

Hotels and resorts typically do not utilize high efficiency motors, pumps, and variable speed drivers, resulting in full load operation even when demand is only partial. This presents a significant opportunity for EE improvements of up to 30% for electrical systems, depending on the pump size and operational needs.

###### Indoor and outdoor Lighting

Hotels with extensive outdoor spaces and decorative lighting systems should consider upgrading to efficient lighting technologies. While some have already made the switch to LED lighting in common areas, others still rely on inefficient options like halogen and metal halide lamps. Overall, there is significant potential for electricity savings through improved lighting applications.

The following table summarises the ECMs that could be applied to the hotels, their expected cost, and saving potential in a conservative manner. Several references were used to estimate the costs; however, two references were very important. The first one is the KfW study in public buildings and the second one, is the EBRD study for hotels. The costs were reviewed and adjusted to the current time. The expected costs are shown in specific investment cost terms. Which means the cost of the ECMs divided by kWh saved per year.

Table 25: Investment cost for ECMs and saving potential for hotels

| ECM | EXPECTED ANNUAL COSTS (EUR/kWh) | SAVINGS POTENTIAL (%) of TEC |
| --- | --- | --- |
| HVAC system replacement | 0.67 | 30% |
| Double glazed windows | 0.27 | 2% |
| Inverter for motors | 0.22 | 1% |
| Upgrade lighting | 0.1 | 5% |
| BMS | 0.3 | 10% |
| PV systems | 0.25 | 1% |
| Solar Water Heaters | 0.34 | 10% |

For the implementation of ECMs in hotels, two scenarios will be assumed, simple renovation which will include upgrading lighting, BMS and installing inverters on motors, and the deep renovation will include all ECMs. The results of implementing the two scenarios are summarized in the following table.

Table 26: Scenario 1 results (hotels)

| SCENARIO 1 (SIMPLE RENOVATION) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Electricity saved (GWh) | Financial savings (EGP) | CAPEX (EUR) | Primary energy savings (GWh) | Co2 savings (tons CO2) | Payback period |
| Lighting | 29.65 | 38,545,000 | 2,965,000 | 76.03 | 12,601 | 2.6 |
| BMS | 47.44 | 61,672,000 | 14,232,000 | 121.64 | 20,162 | 7.8 |
| Inverters on motors | 5.93 | 7,709,000 | 1,304,600 | 15.21 | 2,520 | 5.8 |
| Total | 83.0 | 107,926,000 | 18,501,600 | 213 | 35,284 | 5.8 |

Table 27: Scenario 2 results (hotels)

| SCENARIO 2 (DEEP RENOVATION) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Electricity saved (GWh) | Financial savings (EGP) | CAPEX (EUR) | Primary energy savings (GWh) | CO2 savings (tons CO2) | Payback period |
| Upgrade HVAC System | 59.3 | 77,090,000 | 39,731,000 | 152.1 | 25,203 | 17.5 |
| Replacing windows | 11.9 | 15,418,000 | 3,202,200 | 30.4 | 5,041 | 7.1 |
| Inverters on motors | 5.9 | 7,709,000 | 1,304,600 | 15.2 | 2,520 | 5.8 |
| BMS | 47.4 | 61,672,000 | 14,232,000 | 121.6 | 20,162 | 7.8 |
| Lighting | 29.7 | 38,545,000 | 2,965,000 | 76.0 | 12,601 | 2.6 |
| Solar Water Heater | 59.3 | 77,090,000 | 20,406,176 | 152.1 | 25,203 | 9.0 |
| PV | 5.9 | 7,709,000 | 1,482,500 | 15.2 | 2,520 | 6.5 |
| Total | 219.4 | 285,233,000 | 83,323,476 | 562.6 | 93,249 | 9.9 |

As seen in the Table 26 (that represents the simple renovation – scenario 1), the energy saved represents 14% of the total electricity consumed in this segment (83.0 GWh) while the scenario 2 will achieve 37% of the total electricity consumed in the segment hotels (220 GWh).

#### Office buildings

##### Energy consumption profile

The exact energy consumption for office buildings is estimated according to KfW data set to be 133GWh. It must be noted that some residential buildings are used for administrative activities, and they are billed according to residential electricity tariffs. These buildings are excluded from the above proposed indicator.

There is shortage for audits and benchmark data in administrative building. The report will use data from recent audit carried out by GFA expert for four office buildings in Egypt. The audited administrative building belongs to one entity and located in the same area in the smart village. One of the buildings has a LEED certificate and can be used as a reference value. The following table summarize the consumption and energy intensity for the four buildings.

Table 28: Data from audits of four administrative buildings

| CONSUMPTION CATEGORY | Building 1 | Building 2 | Building 3 | Building 4 |
| --- | --- | --- | --- | --- |
| Electricity consumption (kWh/year) | 3,754,885 | 1,134,707 | 761,916 | 1,102,897 |
| Chilled Water consumption (kWh/year) | 2,774,212 | 2,051,325 | 1,067,521 | 537,482 |
| Solar energy generation (kWh/year) | 19,525 | 31,950 | 134,900 | 71,491 |
| Total Consumption (kWh/year) | 6,548,622 | 3,217,982 | 1,964,337 | 1,711,870 |
| Building area (m2) | 8,040 | 6,832 | 4,167 | 7,740 |
| Energy intensity (kWh/m2) | 815 | 471 | 471 | 221 |

The building four is the LEED certified building and has an energy intensity of 221 kWh/m2. Building two and three has a similar energy intensity of 471 kWh/m2 and building one has the largest value with 815 kWh/m2. By considering building one as a reference, it could be concluded that the potential savings in administrative buildings is between 53% to 72%.

The four buildings do not have any chillers, instead they buy chilled water from district cooling service providers. Chilled water consumption differs between buildings however it is the highest consumer among all applications. The next graph exhibits the Chilled water and electricity intensity for each building.

Figure 14: Chilled water and electricity intensity

The next figure shows the percentage of electricity consumption between different applications in one of the buildings. The lighting is the second largest application that use electricity. HVAC in the graph refers to the electricity consumption for Air Handling Units (AHUs), Fan cool Units (FCUs), and chilled water pumps. Other systems include elevators, city water pumps, computers, etc.

Figure 15: Electricity consumption percentage by application

Several explanations could explain the higher energy consumption in building one. The audit indicated that building one does not have inverters on AHU which makes them run at full load if the load is small. Also, the building has the largest chilled water pump compared to other buildings. In addition, the building has water fountains and outdoor lighting that runs 24/7.

##### Energy conservation measures ECMs

Office buildings have comparable energy profile to hotels. HVAC systems is the largest consumer followed by lighting then other systems such as motors. The same measures applied to the hotels could be applied to office buildings. HVAC systems could be upgraded in most of the office buildings as they depend on split systems or use inefficient technologies, Renovate the buildings to decrease heat transfer, install inverters on pump or motors, install or upgrade BMS, upgrade lightings, install PV and solar heater systems.

The investment costs for each technology assessed are influenced by their utilization, which is directly linked to the load factor that varies depending on the application. For instance, the utilization of energy-efficient technologies decreases proportionally to the load factor in applications with fewer operating hours, such as in office buildings. This results in an increase in specific investment costs that are fixed in EGP/kW. The assumption here is that the working hours in office buildings is half of hotels. The specific investment costs for energy savings in office buildings are shown in the table below.

Table 29: Investment cost for ECMs and saving potential for office buildings

| ECM | EXPECTED ANNUAL COSTS (EUR/kWh) | SAVINGS POTENTIAL (%) of TEC |
| --- | --- | --- |
| HVAC system replacement | 1.34 | 30% |
| Double glazed windows | 0.54 | 2% |
| Inverter for motors | 0.44 | 1% |
| Upgrade lighting | 0.2 | 5% |
| BMS | 0.6 | 10% |
| PV systems | 0.25 | 1% |
| Solar Water Heaters | 0.34 | 10% |

Due to the insufficient data on the costs, savings and other parameters if ECMs is applied in office buildings, the same assumption used in the hotels will be used in administrative buildings. Moreover, the same two scenarios will be assumed. The electricity tariffs will be the assumed according to the highest category in commercial tariff scheme in the year 2024/2025 which is EGP 1.6/ kWh. It is worth noting that is the same tariff that are charged to the four audited buildings. The following two tables summarizes the results from applying ECMs in administrative buildings.

Table 30: Scenario 1 results (commercial office buildings)

| SCENARIO 1 (SIMPLE RENOVATION) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Electricity saved (GWh) | Financial savings (EGP) | CAPEX (EUR) | Primary energy savings (GWh) | Co2 savings (tons CO2) | Payback period |
| Lighting | 6.65 | 10,640,000 | 1,330,000 | 17.05 | 2,826 | 4.3 |
| BMS | 13.30 | 21,280,000 | 7,980,000 | 34.10 | 5,653 | 12.8 |
| Inverters on motors | 1.33 | 2,128,000 | 585,200 | 3.41 | 565 | 9.4 |
| Total | 21 | 34,048,000 | 9,895,200 | 55 | 9,044 | 9.9 |

Table 31: Scenario 2 results (commercial office buildings)

| SCENARIO 2 (DEEP RENOVATION) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Electricity saved (GWh) | Financial savings (EGP) | CAPEX (EUR) | Primary energy savings (GWh) | CO2 savings (tons CO2) | Payback period |
| Upgrade HVAC System | 39.9 | 63,840,000 | 53,466,000 | 102.3 | 16,958 | 28.5 |
| Replacing windows | 2.7 | 4,256,000 | 1,436,400 | 6.8 | 1,131 | 11.5 |
| Inverters on motors | 1.3 | 2,128,000 | 585,200 | 3.4 | 565 | 9.4 |
| BMS | 13.3 | 21,280,000 | 7,980,000 | 34.1 | 5,653 | 12.8 |
| Lighting | 6.7 | 10,640,000 | 1,330,000 | 17.1 | 2,826 | 4.3 |
| Solar Water Heaters | 13.3 | 21,280,000 | 4,576,765 | 34.1 | 5,653 | 7.3 |
| PV | 1.3 | 2,128,000 | 332,500 | 3.4 | 565 | 5.3 |
| Total | 78.5 | 125,552,000 | 69,706,865 | 201.2 | 33,350 | 18.9 |

Few remarks should be noted in this section. First, office building’s electricity consumption could be higher than estimated here. Also, the estimated electricity consumption does not include building that is originally residential but are used for administrative purposes. The payback period in administrative buildings is higher than in hotels due to the lower operation hours compared to the hotels.

#### Other commercial buildings

In this group there are other applications such as private schools, private universities, private hospitals restaurants, and shopping malls. However, no audit data is available at all for these segments. Private hospitals, schools, and universities will be considered the same as public schools, universities, and hospitals in the public building section.

### Summary and market estimation

The next tables summaries the total energy saved, total financial savings and total CAPEX for the two scenarios, simple renovation and deep renovation.

Table 32: Commercial sector results summary for scenario 1 (hotels and office buildings)

| SCENARIO 1 (SIMPLE RENOVATION) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Electricity saved (GWh) | Financial savings (EGP) | CAPEX (EUR) | Primary energy savings (GWh) | CO2 savings (tons CO2) | Payback period |
| Total | 104 | 141,974,000 | 28,396,800 | 267 | 44,328 | - |

Table 33: Commercial sector results summary for scenario 2 (hotels and office buildings)

| SCENARIO 2 (DEEP RENOVATION) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Electricity saved (GWh) | Financial savings (EGP) | CAPEX (EUR) | Primary energy savings (GWh) | Co2 savings (tons CO2) | Payback period |
| Total | 298 | 410,785,000 | 153,030,341 | 764 | 126,599 | - |

The table below presents the market potential for different market shares capture that could be done by the EE Fund for both scenarios.

Table 34: Estimation of market share for commercial sector

| MARKET SHARE (%) | MARKET SHARE (EUR) SCENARIO 1 | MARKET SHARE (EUR) SCENARIO 2 |
| --- | --- | --- |
| 1% | 283,968 | 1,530,303 |
| 2% | 567,936 | 3,060,607 |
| 3% | 851,904 | 4,590,910 |
| 4% | 1,135,872 | 6,121,214 |
| 5% | 1,419,840 | 7,651,517 |

## Public sector

### Sector overview

Even if the public buildings are not among the highest electricity consumer in Egypt, they have huge saving potential between 30% to 50%. In 2021/2022, public buildings consumed 5,879 GWh which represent 3.6% of the total consumption in Egypt. The public building consumption is increasing as seen in the below graph. The consumption is increased by 12% from fiscal year 2017/2018.

Figure 16: Public buildings consumption trend

There are 51,000 subscribers belonging to public buildings. This includes schools, universities, hospitals, administrative buildings, etc. The report will investigate the feasibility in the following buildings:

- Public hospitals

- Public Universities

- Public administrative buildings

- Public schools

#### Hospitals

##### Energy Consumption Profile

Based on the data provided by KfW the total electricity consumption for both public and private hospitals is 958 GWh. Since audit data is available only for public hospital, the report will consider both public and private hospital in this section.

According to CAPMAS statistical book 2021 for health care sector, in 2019, there are 1,782 hospitals with 128,344 beds in public and private hospitals. The number of admissions for the same year. 2019, was 2,032,050 cases in public hospitals.

Energy consumption profile of a hospital is characterized by high energy consumption. That is due the fact that the hospital operates 24/7 unlike other public buildings, that are 8 hours a day, /5 days a week in operation. HVAC consumption is the highest energy consumer in hospitals. For example, in the latest study carried out by Adelphi for KfW, the HVAC consumption in Abu Elrich hospital, a public hospital, represents approximately 75% of the total electricity consumption. This is followed by lighting that consumes around 20%. However, in a study that modelled the energy consumption in a private hospital, the HVAC system consumption was 47% of the total electricity consumption followed by lighting that consumed 36.5% of the total electricity consumption.

The specific energy consumption unit that should be used is in kWh/patient. Bed (kWh/bed). However, the only available data is in kWh/m2. There is one exception, a study that was made in 2011 to develop energy intensity indicators for the Egyptian economy concluded that the energy intensity for public hospital is 7,527 kWh/patient. Bed and 13,590 kWh/patient. Bed for private hospital. This data might be misleading as private hospitals invest more in EEMs/ECMs, however, also provide higher comfort to patients leading to higher consumption.

The KfW study found that the energy intensity for Abu Elrish hospital is approximately 1,050 kWh/m2 and 1029.43 kWh/m2 for El-Dermerdash hospital (another public hospital). This consumption is high compared to international benchmarks. For example, Australia has 460.3 kWh/m2, India 200 kWh/m2, and 550 kWh/m2 for UK.

Implementing EEMs/ECMs in HVAC and Lighting could reduce the electricity consumption significantly. It is expected that HVAC consumption could be reduced by 37% and 70% for lighting.

##### Energy Conservation measures ECMs

Multiple EEMs/ECMs could be implemented to save energy in hospitals in Egypt. The most desirable areas for energy savings are HVAC and lighting. The ECMs will be assumed to be the same as the hotels HVAC upgrade retrofits, lighting upgrades, Building renovation, BMS, Solar heating and PV systems with the same CAPEX. Moreover, the same two scenarios will be assumed, simple renovation and deep renovation.

The savings potential is different than in hotels as the consumption differs. In hospital, fresh air is always used and return air is not accepted for medical reasons. While in hotels, certain percentage of return air is applied and thus save some HVAC energy. The following savings percentages is assumed for ECMs in hospitals.

Table 36: Investment cost for ECMs and saving potential for public hospitals

| ECM | EXPECTED ANNUAL COSTS (EUR/kWh) | SAVINGS POTENTIAL (%) of TEC |
| --- | --- | --- |
| Lighting upgrade | x | 15% |
| HVAC retrofit upgrades | x | 21% |
| Building renovation (double glazed windows) | x | 2% |
| BMS systems | x | 8% |
| Motors and Pumps | x | 1% |
| Solar Water heaters | x | 5% |
| PV systems | x | 1% |

The following table summarizes the savings for applying simple renovation scenarios in hospitals. The total savings from this scenario is 230 GWh which represents 24% of the total hospitals’ consumptions.

Table 36: Public sector results summary for scenario 1 (public hospitals)

| SCENARIO 1 (SIMPLE RENOVATION) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Electricity saved (GWh) | Financial savings (EGP) | CAPEX (EUR) | Primary energy savings (GWh) | CO2 savings (tons CO2) | Payback period |
| Lighting | 143.74 | 229,987,200 | 14,374,200 | 368.57 | 61,090 | 2.1 |
| BMS | 76.66 | 122,659,840 | 22,998,720 | 196.57 | 32,582 | 6.4 |
| Inverters on motors | 9.58 | 15,332,480 | 2,108,216 | 24.57 | 4,073 | 4.7 |
| Total | 230 | 367,979,520 | 39,481,136 | 590 | 97,745 | 3.6 |

The following table summarizes the savings for applying deep renovation scenarios in public hospitals. The total savings from this scenario is 508 GWh which represents 53% of the total hospitals’ consumptions.

Table 37: Public sector results summary for scenario 2 (public hospitals)

| SCENARIO 2 (DEEP RENOVATION) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Electricity saved (GWh) | Financial savings (EGP) | CAPEX (EUR) | Primary energy savings (GWh) | Co2 savings (tons CO2) | Payback period |
| Upgrade HVAC System | 201.2 | 321,982,080 | 134,829,996 | 516.0 | 85,526 | 14.2 |
| Replacing windows | 19.2 | 30,664,960 | 5,174,712 | 49.1 | 8,145 | 5.7 |
| Inverters on motors | 9.6 | 15,332,480 | 2,108,216 | 24.6 | 4,073 | 4.7 |
| BMS | 76.7 | 122,659,840 | 22,998,720 | 196.6 | 32,582 | 6.4 |
| Lighting | 143.7 | 229,987,200 | 14,374,200 | 368.6 | 61,090 | 2.1 |
| Solar water heater | 47.9 | 76,662,400 | 16,488,053 | 122.9 | 20,363 | 7.3 |
| PV | 9.6 | 15,332,480 | 2,395,700 | 24.6 | 4,073 | 5.3 |
| Total | 507.89 | 812,621,440 | 198,369,597 | 1,302.3 | 215,853 | 8.3 |

#### Universities

##### Energy Consumption Profile

According to CAPMAS statistical yearbook 2021, there are 47 universities in Egypt in 2019. Around 26 universities are public and the rest are private universities. KfW data estimates that public universities consumed 583 GWh and private universities consumed 279 GWh last year. The assessment will be done on total consumption.

Two types of buildings exist in universities, buildings without central HVAC and buildings with central HVAC. Lighting is the most energy consumers in universities it could consume between 35% to 45% of total building consumption. HVAC comes second and consumes between 25% to 35% of the total energy consumption. Plug loads such as Computers, printers, and office appliances represent 10% to 15% of total consumption and the rest is for other applications such as water heating. The following figure shows the electricity consumers in two indictive universities.

Figure 17: Energy consumption profile in public universities

##### Energy conservation measures

The investment costs for each technology will depend on its utilization and will be measured in EGP/(kWh*year). This utilization is directly linked to the load factor, which can differ based on the application. For buildings such as universities and administrative offices with fewer operating hours, the utilization of energy-efficient technology also decreases proportionally to the load factor. As a result, the specific investment costs increase as the power-specific investment remains fixed at EGP/kW. The changes in specific investment cost for energy savings will be the same as used for commercial office buildings.

The following table shows the savings for applying simple renovation scenarios in universities. The total savings from this scenario is 207 GWh which represents 24% of the total universities’ consumptions.

Table 38: Public sector results for scenario 1 (universities)

| SCENARIO 1 (SIMPLE RENOVATION) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Electricity saved (GWh) | Financial savings (EGP) | CAPEX (EUR) | Primary energy savings (GWh) | CO2 savings (tons CO2) | Payback period |
| Lighting | 129.31 | 161,634,375 | 25,861,500 | 331.56 | 54,956 | 5.4 |
| BMS | 68.96 | 86,205,000 | 41,378,400 | 176.83 | 29,310 | 16.3 |
| Inverters on motors | 8.62 | 10,775,625 | 3,793,020 | 22.10 | 3,664 | 12.0 |
| Total | 207 | 258,615,000 | 71,032,920 | 530 | 87,929 | 9.3 |

The total savings from this scenario is 353.4 GWh which represents 41% of the total universities’ consumptions.

Table 31: Public sector results for scenario 2 (universities)

| SCENARIO 2 (DEEP RENOVATION) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Electricity saved (GWh) | Financial savings (EGP) | CAPEX (EUR) | Primary energy savings (GWh) | Co2 savings (tons CO2) | Payback period |
| Upgrade HVAC System | 77.6 | 96,980,625 | 103,963,230 | 198.9 | 32,973 | 36.4 |
| Replacing windows | 17.2 | 21,551,250 | 9,310,140 | 44.2 | 7,327 | 14.7 |
| Inverters on motors | 8.6 | 10,775,625 | 3,793,020 | 22.1 | 3,664 | 12.0 |
| BMS | 69.0 | 86,205,000 | 41,378,400 | 176.8 | 29,310 | 16.3 |
| Lighting | 129.3 | 161,634,375 | 25,861,500 | 331.6 | 54,956 | 5.4 |
| Solar water heater | 43.1 | 53,878,125 | 29,309,700 | 110.5 | 18,319 | 18.5 |
| PV | 8.6 | 10,775,625 | 2,155,125 | 22.1 | 3,664 | 6.8 |
| Total | 353.4 | 441,800,625 | 215,771,115 | 906.3 | 150,212.21 | 16.6 |

#### Schools

##### Energy consumption profile

Data about energy consumption in public schools are rare, however, one study published in 2014 assessed energy consumption in public schools. The study of one school could be considered a reference because the Egyptian government established the General Authority of Educational Buildings (GAEB) to design new schools around the country. GAEB uses the same prototype designs to establish schools across many regions of Egypt without consideration to the significant variation in all climatic conditions.

In the new school design, thermal comfort is achieved by cross-ventilation with ceiling fans within the classrooms. This means no HVAC systems or split units are installed in the public schools. This means that the building energy intensity is 1.5 kWh/m2 /year according to the study. Lighting is the main electricity consumer in public schools. However, the study did not indicate the lighting consumption percentage of the total consumption.

According to the records, the school's yearly electricity consumption was 13,019 kWh, which translates to a monthly consumption slightly above 1,000 kWh. Interestingly, this amount is comparable to that of a well-off household. One possible explanation for the low consumption could be the absence of air conditioning in the school. Additionally, the school operates only for a few hours per day, and it remains closed during the three-month summer break.

According to CAPMAS, there are 48,024 public schools. The first estimated that all schools have a comparable consumption and the total school’s annual consumption is 624 GWh that represents 0.38% of the total consumption in Egypt in 2021/2022. However, after data supplied by KfW, the total consumption is 384 GWh.

##### Energy conservation measures

The only ECMs that could be applied is to upgrade lighting in the schools. For the saving calculation the following will be assumed:

- Lighting consumption represent 75% of the total consumption

- 50% of the lamps at the schools are LED and the rest is CFL

- Savings from installing LED lamps is 35%

The below table summarize the savings for upgrading lighting. The total savings from this scenario is 164 GWh which represents 26% of the total schools’ consumptions.

Table 40: Public sector results of lighting upgrades in public schools

| SCENARIO 1 (SIMPLE RENOVATION) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Electricity saved (GWh) | Financial savings (EGP) | CAPEX (EUR) | Primary energy savings (GWh) | CO2 savings (tons CO2) | Payback period |
| Lighting | 100.78 | 125,970,469 | 20,155,275 | 258.40 | 42,830 | 5.4 |

#### Public administrative buildings

#### Energy consumption profile

Based on KfW data, administrative buildings consumed 3,160 GWh which is higher than the calculation based on EgyptERA 2016 report. The explanation for this difference is that in EgyptERA study, administrative buildings have a more limited scope.

Based on the audit submitted and that was done Adelphi consultant, HVAC system is the largest electricity consumer in public building. The below graph shows the electricity consumption percentage based on application in one public building.

Figure 18: Energy consumption profile in public administrative buildings

The electricity consumption profile is similar to the consumption profile in office buildings in commercial sector. Thus, the same assumption will be applied in public administrative buildings.

##### Energy Conservation measures ECMs

With the same assumption applied in office building in commercial sector applied to public administrative buildings, the following table exhibits savings for applying simple renovation scenarios. The total savings from this scenario is 441 GWh which represents 14% of the total universities’ consumptions.

Table 41: Public sector results of Scenario 1 (administrative buildings)

| SCENARIO 1 (SIMPLE RENOVATION) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Electricity saved (GWh) | Financial savings (EGP) | CAPEX (EUR) | Primary energy savings (GWh) | CO2 savings (tons CO2) | Payback period |
| Lighting | 157.60 | 252,167,202 | 31,520,900 | 404.11 | 66,982 | 4.3 |
| BMS | 252.17 | 403,467,523 | 151,300,321 | 646.58 | 107,171 | 12.8 |
| Inverters on motors | 31.52 | 50,433,440 | 13,869,196 | 80.82 | 13,396 | 9.4 |
| Total | 441 | 706,068,166 | 196,690,418 | 1,132 | 187,549 | 9.5 |

The following table exhibits savings for applying renovation more ambitious renovation scenario. The total savings from this scenario is 1,481 GWh which represents 47% of the total universities’ consumptions.

Table 42: Public sector results of scenario 2 (administrative buildings)

| SCENARIO 2 (DEEP RENOVATION) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Electricity saved (GWh) | Financial savings (EGP) | CAPEX (EUR) | Primary energy savings (GWh) | Co2 savings (tons CO2) | Payback period |
| Upgrade HVAC System | 630.42 | 1,008,668,808 | 844,760,127 | 1,616.46 | 267,928 | 28.5 |
| Replacing windows | 63.04 | 100,866,881 | 34,042,572 | 161.65 | 26,793 | 11.5 |
| Inverters on motors | 31.52 | 50,433,440 | 13,869,196 | 80.82 | 13,396 | 9.4 |
| BMS | 252.17 | 403,467,523 | 151,300,321 | 646.58 | 107,171 | 12.8 |
| Lighting | 157.60 | 252,167,202 | 31,520,900 | 404.11 | 66,982 | 4.3 |
| Solar water heater | 315.21 | 504,334,404 | 108,468,980 | 808.23 | 133,964 | 7.3 |
| PV | 31.52 | 50,433,440 | 7,880,225 | 80.82 | 13,396 | 5.3 |
| Total | 1,481 | 2,370,371,699 | 1,191,842,322 | 3,799 | 629,630 | 17.1 |

#### Public sector summary

The next tables summaries the total potential for energy savings, total financial savings and total CAPEX for both scenarios, simple renovation and deep renovation.

Table 43: Public sector results summary for scenario 1

| SCENARIO 1 (SIMPLE RENOVATION) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Electricity saved (GWh) | Financial savings (EGP) | CAPEX (EUR) | Primary energy savings (GWh) | CO2 savings (tons CO2) | Payback period |
| Total | 979 | 1,458,633,154 | 327,359,749 | 2,510 | 378,851 | - |

Table 44: Public sector results summary for scenario 2

| SCENARIO 2 (DEEP RENOVATION) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Electricity saved (GWh) | Financial savings (EGP) | CAPEX (EUR) | Primary energy savings (GWh) | CO2 savings (tons CO2) | Payback period |
| Total | 2,444 | 3,750,764,233 | 1,626,138,309 | 6,266 | 1,038,525 | - |

The below table summarize the market share (public sector) for different percentage of market capture that could be done by the EE Fund in both scenarios.

Table 45: Estimation of market share for public sector

| MARKET SHARE (%) | MARKET SHARE (EUR) SCENARIO 1 | MARKET SHARE (EUR) SCENARIO 2 |
| --- | --- | --- |
| 1% | 3,273,597 | 16,261,383 |
| 2% | 6,547,195 | 32,522,766 |
| 3% | 9,820,792 | 48,784,149 |
| 4% | 13,094,390 | 65,045,532 |
| 5% | 16,367,987 | 81,306,915 |

# Market Supply Side assessment

## EE Market Actors

### EE service providers

Implementing ECMs after performing audits requires equipment, machinery, and skilled worker for proper installation. Although the number of working ESCOs in Egypt is not known, supplier, EPC contractors, and energy companies provides EE equipment.

In the Egyptian market, only a handful of companies possess the resources, capabilities, and expertise to offer comprehensive ESCO services, including EE project implementation. As for the majority of the energy service providers, even the multinational ones, they limit themselves to providing EE products and energy supply equipment like diesel generators for electricity generation.

For example, several cogeneration projects were established by engineering, procurement and contracting (EPC) companies to sell electricity and chilled water/hot water/steam. Some other distributors of motors inverters sell inverters as EE products. Moreover, companies that sell building management systems (BMS) or SCADA systems market their products to save energy.

In other words, equipment distributors promote their products as EE equipment’s and present themselves as EE suppliers. For example, boiler suppliers will sell boiler economizer, steam traps, and condensate pumps as EE products in some cases. However, ESCOs services is not well known in Egypt. Below is the history of ESCOs in Egypt.

The ESCO industry in Egypt had its origins in the Energy Efficiency Improvements and Greenhouse Gas Reductions (EEIGGR) GEF project, which was implemented between 1998 and 2006. The project was granted USD 4.11 million to address technical, institutional, and capacity obstacles to energy efficiency in Egypt. As a result of this initiative, eight ESCOs were established to offer energy efficiency advice and financing.

The project provided comprehensive training to these ESCOs in energy auditing, EE technologies, project evaluation, and financing. Additionally, a sales process was developed for both private and public sectors. In 2004, the ESCO support system was fine-tuned to focus on a supplier-based credit model with uncomplicated contracts and low-risk technologies with short payback periods. This project also included the creation of a loan guarantee program for ESCOs to facilitate their access to bank financing. However, the program's success remains unclear due to insufficient information.

The project supported ESCOs by providing them with financial guarantees to obtain loans from banks. The program selected seven ESCOs and implemented energy-efficient technologies to achieve a payback in under three years. From 2004 to 2010, 46 projects were completed at a cost of L.E. 68 million, with a guarantee of LE 30 million. A diagram of the loan guarantee program is shown in Figure 11. The loan guarantee scheme was designed and set up accordingly.

Figure 19: Loan guarantee program design, Source UNIDO

Besides the above project to create ESCO market several other projects had the same objective. Below are brief of the ESCO initiatives

- The Credit Guarantee Company (CGC) projects

A joint-stock company in Egypt, privately owned by 9 local banks and working with around 30 others, offers a guarantee for a certain percentage of loans and credit facilities given by these banks to Small and Medium-sized Enterprises (SMEs) without requiring any collateral. Here are some of the key results and accomplishments of the initiative:

  - A total of 37 projects focused on energy efficiency and reducing greenhouse emissions were carried out through a loan guarantee program. The total loan amount was EGP 46.25 million, with EGP 15.37 million being guaranteed

  - Seven ESCOs were involved in executing EE projects, but unfortunately, none of them were able to sustain their operations

  - In their EE projects, both USAID and UNDP have made efforts to promote ESCOs in Egypt. The objective of these projects was to establish a business model that would demonstrate how ESCOs can assist clients in securing financing from commercial banks. Ultimately, the goal was to create a self-sustaining market for ESCO activity

- The UNDP/GEF project “Improving Energy Efficiency for Lighting & Building Appliances”

In 2013, the Ministry of Electricity and Energy (MEE) collaborated with the UNDP and the GEF to launch a project under the Supreme Energy Council (SEC) Energy Efficiency Programs. The project aims to decrease electricity consumption for residential lighting and minimize greenhouse gas emissions. Lighting was selected as it accounts for over 40% of total electricity consumed by households. Existing ESCOs participated in implementing the project, with the Loan Guarantee Mechanism being a finance source.

- The USAID Egyptian Environmental Policy Program (EEPP)

The EEPP was financed by the USAID and administrated by the Egyptian Energy Service Business Association (EESBA). One of the main important achievements and outcomes of the project implementation was the training and certification of 300 Energy Managers since the year 2000.

ESCOs' business in Egypt owes its start and establishment to the implementation of projects such as the EEIGGR project in the late 1990s. Despite the energy efficiency investment, technical and financial support provided by GEF, USAID, and UNDP, the ESCOs energy efficiency business opportunities are limited, and the industry and market development are still at an early stage. This is due to the absence of necessary market development drivers and the existence of several barriers hindering ESCOs' EE business and market development.

### EE Experts

Egypt has a large number of qualified energy consultants and auditors who have received training and accreditation from various donor-funded programs. These include Energy Management System Training (ISO 50001) by UNIDO, Certified Energy Management Professional Program by RCREEE, EUREM Energy Manager by Arab-German Chamber of Commerce, as well as individuals accredited under the American Association of Energy Engineers and the Green building rating systems, particularly Leadership in Energy and Environmental Design (LEED) by the US Green Building Council. Below are the available programs in Egypt to train individuals to be certified in EE field.

Table 38: EE available programs in Egypt

| UNIDO SHIP Program | The SHIP program targeted energy experts and engineers from industrial sector. candidates joined a 5-days training, on various thermal energy efficiency assessment tools. Afterwards competent candidates joined an on-job training program for a period of 6 months, where they join the project technical team to develop a number of energy audits in the industrial sector. Afterwards, candidates joined another 5-days training on SWH systems design and feasibility. Afterwards competent candidates will join an on-job training program for a period of 6 months, where they join the project technical team to develop a number of studies in the industrial sector. Training Programs – SHIP (shipprojectegypt.org) |
| --- | --- |
| UNIDO IMEEP program | The program trained around 80 Experts on energy audits for motors, compressors, and pumps. After the training, the experts carried out collectively more than 100 energy audits in Egyptian factories. The program currently preparing to train experts on M&V to up take ESCO market Home - Industrial Motors Energy Efficiency Programme (imeep-eg.org) |
| PA-CEMP | The Pan Arab Certified Energy Management Professionals program (PA-CEMP) is an in-depth professional certification program for Energy Mangers tailored for the Arab region. PA-CEMP aims to assist EE policy makers and program administrators in planning and implementing Energy Efficiency programs and measures in the Arab region. Through a recognized certificate, energy management professionals will be equipped with the latest technologies, energy essentials and the effective means with solutions to reduce energy consumption in a cost-effective approach and achieve energy targets in line with the Arab EE Guideline. In addition, the program will support boosting job creation in the Arab region by reducing energy imports and delivering cost effectively measures. PA-CEMP is designed for a wide target audience. It will enable all its participates which varies in their educational levels, years of experience and background will enabled all to create a commercially viable Energy Concept. This helps organizations to directly apply what the participants learnt in the program and to use energy efficiently and save money. About PA-CEMP – RCREEE |
| EUREM | EUREM is a standardized training of further education that enhances the skills of technical experts in the field of energy efficiency improvement. The qualification of an employee as an Energy Manager creates operational know-how in order to continuously uncover energy-related weaknesses, to use savings potential, and to optimize energy efficiency. The project work achieved saves companies an average of about 30,000 euros per year in energy costs. Training & Qualification (ahk.de) |
| CEM | Certified Energy Manager (CEM) is designed to help energy professionals, including energy managers, energy engineers, facility and business managers, industrial engineers, supply chain professionals, utility officials, consultants, contractors, financial officers, and energy service company (ESCO) professionals become more aware of and effective at identifying and implementing the best energy management strategies. CEM – Tarsheed (tarsheedec.com) |

### EE Suppliers and Retailers

Below is a summary of existing suppliers that could supply EE equipment according to ECMs

- Efficient Appliances

for inverter AC, besides importing efficient AC, several local suppliers exist like Toshiba El Araby and Unionair. For efficient refrigerators, all refrigerators above A efficiency class are imported. LG, Samsung, and Bosch distributors sell up to A+++ efficiency class.

- HVAC equipment

Several vendors supply EE equipment in HVAC such as VRV systems, heat recovery for AHU, and effect chillers. ECS, Korra energy, and Tiba group, and other supplies these services

- Steam system

Equipment such as economizers, blow down heat exchanger are available largely through importation. Handful local suppliers supply part of required equipment

- Motors systems (pumps, compressors, and motors)

EE equipment such as inverters, soft starters are imported through dealers to the market. Several well-known companies have their distributors in Egypt such as ABB, Schneider Electric, and Emerson.

- Solar PV

Distributor for international PV panel, inverters, and DC cables supply the whole market. There were few local assembly activities for PV panel but are not available now.

- Solar water heaters

Solar water heaters are imported from Europe or China through distributors or traders. There were some trials to manufacture some components of the solar water heaters (e.g., tanks) however, nothing was achieved till now.

- Efficient lighting

Several manufacturers exist in Egypt for LED lamps such as Al Araby Group, 3Brothers, and Elios. Also, several international brands are available through importation.

### Regional DISCOs

Egypt has nine DISCOs covering all electricity needs in Egypt. On fiscal year 2021/2022, DISCOs sold 81.4% of all electricity in Egypt. The transmission company sold the rest on a high voltage Level. Canal Distribution company sold 18.7% of the total electricity sold, followed by South Cairo, North Cairo, and Middle Egypt with shares of 16%, 13.4% and 12.1%, respectively. The next figure shows the share of electricity sold by each distribution company.

Figure 19: Sold electricity % by DISCOs, Source: EEHC

South Cairo has the largest number of subscribers with percentage of 16% followed by South Delta and North Cairo having shares of 13.1% and 12.6%, respectively. The next figure exhibits the shares of number of subscribers by each distribution company.

Figure 20: Shares of NO. of subscriber, Source: EEHC

The difference between purchased electricity and sold electricity could be significant. For example, In South Cairo distribution company, the percentage of purchased electricity to sold electricity is 68.6% indicating 31.4% losses. Upper Egypt Distribution Company has the second largest losses with 22.2% percentage. The below graph shows the sold and purchased electricity and the ratio of sold electricity to purchased electricity.

Figure 21: Sold and purchased electricity by each DISCOs

Under the new electricity Law 87/2015, DISCOs are required to formulate an annual plan that will be reviewed by the regulator (EgyptERA) detailing how the distribution company plans to address the following areas:

- Managing electricity demand

- Improving EE

- Encouraging the use of RE

- Raising awareness about EE

An EE project was implemented in three DISCOs (North Cairo- Alexandria- North Delta) to reduce electricity network losses. The project was financed by JICA (USD 843.258 million) to establish control centres and advanced communication systems to ensure energy availability during peak times, to apply automatic cut-off and connection commands, and to reduce outage periods. The following table represent the results of this project and the reduction in technical and commercial losses.

Table 47: Reduction of losses after project implementation

| DISCOs | Tech losses before | Tech losses after | Comm losses before | Comm losses after |
| --- | --- | --- | --- | --- |
| Alexandria | 3.18% | 2.52% | 6.7% | 4.9% |
| North Delta | 5% | 4.5% | 5.86% | 3.65% |
| North Cairo | 5.91% | 5.15% | 5.85% | 6.51% |

## Available financing Mechanism

In the below table, a summary of available financing facilities for EE project.

Table 48: Available financing facilities in Egypt

| Program | Program Scope | Targeted Sector | Source of finance | Structure of fund | Incentives | Participating banks | Technical assistance | Fund Size |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EBRD – GEFF II | Resource efficiency | Commercial, Industrial, agriculture | EBRD – AFD | offer a loan option at the market rate, along with a 10% grant that is given back to the client as a rebate once the system is successfully installed and functioning. | 10% grant (could be 15% depending on technology) | QNB, Arab African Bank, Alex Bank, National Bank of Kuwait | Assistance to bank | 175 million dollars |
| Eco-FEI – EU | Renewable energy, energy efficiency and environmental compliance, cleaner production | Industrial | EU | Low interest loans are provided for a maximum of 10 million EGP | Subsidized interest (3.5%) | NBE | Assistance to client | 30 million dollars |
| EBRD – VCFF | Resource efficiency and technology modernization | Industrial and commercial sector (SMEs by EU definition) | EBRD | Loan at market rate with a grant component) | Likely to be a grant of 10% | QNB, others to be included | Assistance to bank | 82 million dollars |
| EBRD – NBE | Renewable energy and energy efficiency | SMEs in general by EU definition | EBRD | Loan at market rate | None | NBE | Assistance to bank | 117 million dollars |
| EPAP III | Environmental and pollution control | Industrial | KfW | Loan at reduced rate | Lower interest | CIB, NBE, QNB | Assistance to bank | 140 million dollars |
| MSMEDA | World Bank Renewable and energy efficiency | Industrial, agriculture and commercial | WB | Loan at reduced rate | Lower interest | TBD | None | 50 million Dollars |

## Gap analysis and Identified needs for technical assistance

From the market assessment of demand side and supply side, several gaps exist. These gaps belong to different aspects such as regulatory, technical, policy and financial. A summary of gaps that are identified are below.

- The lack of awareness about ESCOs model in Egypt. There is a lack of awareness among various stakeholders, including customers, about the financing schemes available, as well as banks and financing institutions' understanding of the role of ESCOs in achieving energy efficiency targets

- absence of relevant databases in addition to information dissemination. The team experienced difficulties in collecting information about energy consumption data such as specific energy consumption, energy consumption profiles, energy audits, etc. Moreover, most of the available data are scattered and collected from different sources. In addition, most of data are old and not updated on regular basis

- The non-existence of M&V services. Measurement & Verification of energy consumption services by certified 3rd parties do not exist in Egypt in case of any dispute between ESCOs beneficiaries

- One of the main challenges facing ESCOs is the absence of suitable financing options and incentives. Additionally, banks often lack the necessary expertise to evaluate ESCO projects, which they often consider to be high-risk ventures

- The lack of proper regulations and institutional framework hinders the ability of Egyptian ESCOs to carry out Energy Performance Contracts in the current market conditions

From the above points, technical assistance will be required for

- Set up the required regulatory framework and train institution to be able to implement ESCOs business models

- Set up and train third party entities to offer M&V services in the market

- Train banks on how to assess and evaluate the risks of an EE project and ESCO/EPC model concept

## Lessons learned, cooperation opportunities with other schemes and institutions

### Lessons learned

When assessing the market supply and demand for ESCOs and EE providers, there are several key factors that can be identified as necessary for promoting growth in these markets. Below are some points thar are concluded from the assessment.

- Electricity prices are still low, and the subsidy provided to electricity consumer will act as a barrier. from market demand assessment we have seen long payback periods could be over 20 years for ECMs that could be applied. This will discourage entities to invest in EE products

- The team faced difficulties in collecting data on energy consumption data for different applications, consumption profiles, impact of implementing ECMs, and audit case studies in commercial and public sector. A comprehensive data base should be developed for each building type. this will help the future ESCOs and EE provider to quickly estimate energy saving, implementation cost costs, and ECMs performance expectations

- The market still misses the required regulatory framework to pave the way for ESCOs model implementation. Moreover, the governing body is still not identified. If ESCOs model to be developed, adequate legal framework should be laid out

- In the ESCOs market, there is a noticeable absence of M&V services. Egypt lacks official and accredited M&V service providers in the energy sector, and there is limited knowledge of best practices in this area

- EE Standards and codes need to be enforced. As seen, building codes was developed and implemented, however, it is not enforced and followed. This results in increased energy consumption and reduce the EE market potential. It must be noted that enforcing standards and codes require develop the capacity or implementation bodies which are currently miss the skills and knowledge

- Implementing EE in public building is not an easy task and involves a bureaucratic process. This rigid and inflexible procedures need to be updated and simplified for accelerated ECMs implementation

### Cooperation opportunities

Two cooperation opportunities were identified during data collection for this project. One with African development bank (AfDB) and the other one the United Nation for Industrial Development (UNIDO).

- African development bank (AfDB)

Currently AfDB is studying supporting the establishment of EE programs such as replacing one million refrigerators. Since this is currently on-going study KfW and AfDB could join forces and develop one program or finance the replacing program including other appliances such as AC that has better results and faster payback period.

- United Nation for Industrial Development (UNIDO)

UNIDO is currently implementing a project to drive ESCOs market in Egypt. Although the program is part of industrial EE in motors that indicate the efforts will be dedicated to industrial sector, nevertheless, the program impact extends to the ESCOs market.

UNIDO issued an EOI to EE companies that are interested to participate in a capacity development program and to be qualified to operate as ESCOs. Moreover, in the next few months, UNIDO will implement a program for M&V capacity development for qualified individuals who has the basics to join this program. The experts who got certified previously from UNIDO to deliver energy audits will be invited to join this capacity development program.

In addition, UNIDO will train selected individuals for ESCOs dispute arbitration. The only entity that can carry out arbitration is the commercial arbitration center. Unfortunately, they do not have the required expertise to carry out arbitration for ESCOs disputes. UNIDO is in a mission to develop the required skills and knowledge to be able to carry out this task.

# Pipeline development

When it comes to EE projects, there are various factors that contribute to the growth of the market. However, there are also several obstacles that need to be addressed, as they have the potential to jeopardize the entire EE industry.

## Drivers

- Strong base of EE experts who has extensive experience in performing audits and implementing projects related to energy efficiency. EE experts were trained through various international projects and have accumulated a wealth of knowledge and expertise over time

- The government intends to remove subsidies for energy prices and set a plan for electricity price increase and subsidy phase out or removal. However, continuously change in exchange rate undermines these efforts and let to change in subsidy removal plans several times

- The government has implemented effective legislation, such as the electricity law, to improve EE. The law mandates that various stakeholders prepare plans to promote EE

- Strong support from international DPs such as KfW, UNIDO, EBRD, IFC and GIZ

## Barriers

- There are various uncertainties that may impact the feasibility of EE projects, such as changes in energy prices and exchange rates. These factors can greatly affect the costs involved and make it difficult to determine the viability of such projects

- Prospective clients and lenders often have limited knowledge of ESCOs and are concerned about the reliability of achieving energy savings through EEMs. It can be challenging for them to understand what ESCOs do and the and how they will collect their savings

- The lack of a reliable M&V market poses a significant challenge for ESCO companies. When it comes to energy services in Egypt, there are no official or accredited M&V service providers, and the best practices for such services are not widely known or implemented

- No data collection or analysis to be referenced for M&V services or for setting the consumption baseline when doing audits

- Although there are building codes that specify the energy performance of buildings, however it is not enforced till now

- One of the major challenges faced within the ESCO industry is the shortage of human resources with expertise in financing and marketing. Additionally, prospective clients often lack the technical know-how and experience required to accurately evaluate the advantages and drawbacks of participating in ESCO projects.

- The EPC business model poses several challenges, such as narrow profit margins, uncertainties related to remuneration based on guaranteed savings, risks associated with non-compliant clients, and the expenses associated with preparing intricate bids that may not be accepted

- It can be challenging to secure necessary financing from banks due to their perception of high risk in lending to Energy Service Companies (ESCOs), especially when the ESCO is the borrower. Banks consider various risks, including the ESCO's reputation, their ability to access the end-user facility, and potential conflicts between the ESCO and end-user due to a lack of clear mechanisms to verify savings

## Selection Criteria

Several selection criteria could be laid to ensure selecting the optimum ECMs for EE Fund pipeline. Selection criteria could be based on financial indictors or other indicators such impact or emission savings. Below are the proposed indictors that could be implemented for ECMs selection.

- Simple Payback Period

The payback period is the duration required to recoup the investment cost. In other words, it is the time taken for an investment to break even. Shorter payback periods indicate better investments, while longer ones are less appealing. However, a drawback of the payback period is that it does not consider the time value of money. The unit of this selection criteria will be in years.

A threshold needs to be established for this selection criteria. For example, projects with payback periods that is longer than – for example – 10 years should be discarded. This ensures the sustainability of the project and targets the low hanging fruit project first.

Projects that have low payback period are replacing AC in the residential sector, lighting upgrades, BMS in commercial and public sector.

- Primary energy saved

Making energy efficiency improvements is an effective way to decrease the amount of energy needed to provide a service. Energy savings are essential to reap multiple benefits associated with energy efficiency, including economic, social, and environmental advantages. The primary energy consists of consumption and conversion losses during energy transformations, so primary energy saving is crucial to reduce energy conversion and transformation. The unit of this selection criteria could be in kWh or toe or any other energy unit.

A proposed value of 15% savings from the base line could be implemented. The projects that have a primary energy savings of 15% or more will be selected to be implemented. Primary energy savings were selected because some applications incur small savings in the final energy use and more savings in primary energy. An example of that, when upgrading HVAC systems and install absorption chillers, low saving, or even no savings, is achieved in final energy consumption but savings is achieved in primary energy.

- Emission savings

Improving energy efficiency has a variety of positive impacts on the environment. It significantly lowers greenhouse gas emissions, including direct emissions from fossil fuel usage as well as indirect emissions from electricity production. This is an important step in reducing our carbon footprint. The unit of this selection criteria will be in tons of CO2e.

Usually, emissions savings are directly linked to energy savings. However, some emission could be achieved from implementing EE measures. As an example, implementing a replacing program for AC could lead to emissions savings other than emissions from reducing energy consumptions. Changing the refrigerant used in AC with more environmentally friendly refrigerant could increase the impact of ECMs implementation.

Tariff groups according to monthly consumption

MoE, 2022. Egypt's First Updated Nationally Determined Contributions, Cairo: Ministry of Environment

MoE, EEAA, 2018. First Biennial Update Report, Cairo: Ministry of Environment, Egyptian Environmental Affairs Agency

Ahmed A. Hassan, Norhan El Dallal, Jan Grözinger, Nesen Surmeli-Anac, Cooling Sector Status Report Egypt, Guidehouse,2022

EEHC, Annual Report 2021/2022, Egyptian Electricity Holding Company, 2023

EEHC, Annual Report 2021/2022, Egyptian Electricity Holding Company, 2023

Law No. 196 of 2017 for Gas Market Activities Regulation – Policies - IEA

meetMED, Energy Efficiency in Buildings, 2020

RCREEE, Country Paper on EE in Buildings

meetMED, Energy Efficiency in Buildings, 2020

EEHC, Annual Report 2021/2022, Egyptian Electricity Holding Company, 2023

Ahmed A. Hassan, Norhan El Dallal, Jan Grözinger, Nesen Surmeli-Anac, Cooling Sector Status Report Egypt, Guidehouse,2022

EEHC, Annual Report 2021/2022, Egyptian Electricity Holding Company, 2023

Second National Energy Efficiency Action Plan (NEEAP)

Ali Kabdel-Rahman, Ahmed H. Ali, Shinichi Ookawara, An Analysis of Thermal Comfort and Energy Consumption within Public Primary Schools in Egypt, 2014

Typology 1 is a block of base 25m x 11m x 18m with a 2.3:1 aspect ratio. The total area of one apartment is 122 m2 with a net conditioned area of 60 m2, representing three rooms per apartment. Windows are single-glazed, transparent, and have a 0.003m thick glass pane. The total amount of glass in the North and South facades is estimated to be between 45% and 35% of the total wall area. Typology 2 is a twelve-story building block of a base 30m x 20m x 34m with a 1.5:1 aspect ratio. The building’s gross floor area is 7200 m2, and the net conditioned area is 60 m2 representing three rooms per apartment. The building has the same construction properties as Typology 1. The amount of glass used was estimated at 46% in the short façades and 20% in the longer facades of the total wall area.

Shady Attia, Arnaud Everard, Benchmark Models for Air-Conditioned Residential Buildings in Hot Humaid Climate, 2013

The World Bank, Energy Efficiency and Rooftop Solar PV Opportunities, 2017

Ministry of Trade estimated that only 28,000,000 households are occupied

After deducting the number of unoccupied households

Ahmed A. Hassan, Norhan El Dallal, Jan Grözinger, Nesen Surmeli-Anac, Cooling Sector Status Report Egypt, Guidehouse,2022

Ahmed A. Hassan, Norhan El Dallal, Jan Grözinger, Nesen Surmeli-Anac, Cooling Sector Status Report Egypt, Guidehouse,2022

Ahmed A. Hassan, Norhan El Dallal, Jan Grözinger, Nesen Surmeli-Anac, Cooling Sector Status Report Egypt, Guidehouse,2022

Extracted from JICA study and by dividing the total Ac consumption on the estimated number of ACs

https://www.elarabygroup.com/en/air-conditioners-coolers-and-purifiers/air-conditioners/2-25-hp

Assuming an exchange rate of 34

https://www.elarabygroup.com/en/tornado-warm-fluorescent-lamp-19-watt-60-cm-yellow-fl20t9-19ww-t?gclid=CjwKCAjw4ZWkBhA4EiwAVJXwqWpSHbkyeXe7WnH5zIwitSibjStwMJIAnt5ZwzhAkhnVE9OrzWIZrRoC9FUQAvD_BwE

https://www.elarabygroup.com/en/tornado-daylight-tube-led-lamp-9-watt-60-cm-white-light-tm-d10h

CAPMAS, The annual Bulletin for industrial commodities production for private sector, 2023

https://www.elarabygroup.com/en/tornado-refrigerator-advanced-no-frost-275-liter-silver-rf-275vt-sl

https://www.lg.com/eg_en/refrigerators/lg-gtf402ssan

Buy online Best price of LG GTF402SSAN Top Mount Refrigerator 401L Silver in Egypt 2020 | Sharafdg.com

CAPMAS, The annual Bulletin for industrial commodities production for private sector, 2023

Second National Energy Efficiency Action Plan (NEEAP)

https://rb.gy/dvmyo

39% conversion rate was used as indicated in 2023 energy balance

Does not include mixed-used buildings

CAPMAS, 2019

The World Bank, Energy Efficiency and Rooftop Solar PV Opportunities, 2017

KfW estimations for 2022

Statistical Book 2021, Tourism, CAPMAS

Sustainable energy opportunities in the Hotel Sector of Egypt, 2013

The audit was done under a project financed by one of international development institution

Sustainable energy opportunities in the Hotel Sector od Egypt, 2013

This study will be introduced later in the public building section

Total Energy Consumption

Total Energy Consumption

The World Bank, Energy Efficiency and Rooftop Solar PV Opportunities, 2017

EEHC annual report 2021/2022

Blattmann, K., Schröder, D., Hercegfi, A., Elfarra, K. (2016): Energy Efficiency in Public Buildings in Egypt – Update 2016. Berlin: adelphi

Micheal A. William, Aly M. El-Haridi, Ahmed A. Hanafy, and Abd El-Hamid A. El-Sayed, Assessing the Energy Efficiency improvement for hospitals in Egypt using building simulation modeling, Engineering Research Journal, Vol. 42, No. 1, January 2019, PP: 21-34, Faculty of Engineering, Menoufia University, Egypt

Bed refers to the number of days that a bed is occupied. If a patient leaves a hospital on a particular day, and another patient is allocated that bed, the number of bed days is counted as two. It is an indicator of the occupancy rate of a hospital, compared to its total bed availability

Environics, Development of a System of Energy Intensity Indicators for the Egyptian Economy, UNEP

Miller, Wendy & Liu, Aaron & Crompton, Glenn & Ma, Yunlong. (2020). Healthcare Sector Energy Baseline and Key Performance Indicators. 10.13140/RG.2.2.32475.28960.

TEC – Total Energy Consumption

Abdel-Rady, Ahmed & Abdel-Rahman, Ali & Ali, Prof. Dr. Eng, Ahmed Hamza H. & Ookawara, Shinichi. (2014). An Analysis of Thermal Comfort and Energy Consumption within Public Primary Schools in Egypt.

Abdel-Rady, Ahmed & Abdel-Rahman, Ali & Ali, Prof. Dr. Eng, Ahmed Hamza H. & Ookawara, Shinichi. (2014). An Analysis of Thermal Comfort and Energy Consumption within Public Primary Schools in Egypt.

Abdel-Rady, Ahmed & Abdel-Rahman, Ali & Ali, Prof. Dr. Eng, Ahmed Hamza H. & Ookawara, Shinichi. (2014). An Analysis of Thermal Comfort and Energy Consumption within Public Primary Schools in Egypt.

This number includes preprimary, primary, preparatory, and secondary schools and does not include Azhari or special schools.

ESCO market assessment, UNIDO, 2022

ESCO market assessment, UNIDO, 2022

Annual Report 2021/2022, EEHC

ESCO market assessment, UNIDO, 2022

https://www.afdb.org/en/news-and-events/african-development-bank-invests-energy-efficiency-central-pillar-energy-transition-61163